# AIA CAD Layer Guidelines

## TABLE OF CONTENTS

Key:  = Section contains a downloadable Microsoft Excel document

  - Introduction

  - Overview

  - A Brief History of the CAD Layer Guidelines

  - Layer Name Format

  - Hierarchy of Data Fields

  - Before You Begin

  - Discipline Designator, Level 1

  - Discipline Designator, Level 2

  - Major Group

  - Minor Group

  - Status (Phase)

  - Drawing View Layer List

  - Drawing View Field Codes

  - Drawing View Layer Names

  - Annotation Layer List

  - Annotation Field Codes

  - Annotation Layer Names

  - Appendix A - List of Discipline Designators, Major and Minor Groups, and Status Fields

  - Discipline Designators

  - Major Groups

  - Minor Groups

  - Status Fields

  - Appendix B - Common Layer Lists by Discipline

  - Architectural Layer List

  - Civil Layer List

  - Contractor/Shop Drawing Layer List

  - Electrical Layer List

  - Distributed Energy Layer List

  - Equipment Layer List

  - Fire Protection Layer List

  - General Layer List

  - Geotechnical Layer List

  - Hazardous Materials Layer List

  - Interiors Layer List

  - Landscape Layer List

  - Mechanical Layer List

  - Operations Layer List

  - Plumbing Layer List

  - Process Layer List

  - Resource Layer List

  - Structural Layer List

  - Survey/Mapping Layer List

  - Telecommunications Layer List

  - Other Disciplines Layer List

  - Appendix C - Complying with NCS and ISO 13567

  - Overview

  - Field Codes

  - Field Codes and Language

  - ISO 13567 Conformance

  - Field Names and Definitions

  - "Discipline Designator" vs. "Agent Responsible"

  - "Agent Responsible" and Professional Liability

  - "Discipline Designator" and the Building Life Cycle

  - "Discipline Designator" and ISO 13567 Conformance

  - Field Code Restrictions

  - NCS and ISO 13567 Implementation Options

  - NCS and ISO 13567 Implementation Guidelines

# Introduction

## OVERVIEW

Virtually all vector-based CAD systems support the concept of layers. This function allows building design information to be organized in a systematic fashion, facilitates the visual display of the information on a computer screen, and allows the information to be efficiently converted to the conventional print media of drawings. Efficient use of layers can reduce document preparation time and improve document coordination. Organizing data by layers allows a single CAD file to contain a wealth of information about a building or facility. By turning selected layers on or off, data can be created, reviewed and edited according to a hierarchy that simulates the physical organization of building systems, the relative position of building elements, or the sequence of construction.

## A BRIEF HISTORY OF CAD LAYER GUIDELINES (CLG)

The American Institute of Architects (AIA) published the first edition of CAD Layer Guidelines in 1990. The early success of the first edition and rapidly evolving technology resulted in the second edition being published in 1997. The most significant change between the first and second editions was the elimination of the "short" layer name

format and the adoption of the long layer name format as a single standard. The second edition also included additional layer field codes for remodeling projects, added new discipline designations for interiors, telecommunications, and other disciplines, and improved the method of organizing drawing annotation.

In July 1997, the AIA agreed to incorporate CAD Layer Guidelines into the emerging United States National CAD Standard® (NCS), a project of the National Institute of Building Sciences (NIBS). The AIA and NIBS were joined in that effort by the Construction Specifications Institute (CSI) and what is now known as the CADD/GIS Technology Center of the U.S. Army Corps of Engineers. CSI and CADD/GIS Technology Center agreed to incorporate their own publications, the Uniform Drawing System and the Plotting Guidelines, respectively, into the NCS. These four constituent publishers, as they came to be known, were joined by a number of building design and construction industry organizations in developing and publishing the NCS.

In March 1999, the U.S. National CAD Standard Project Committee (NCS Project Committee) formally accepted CAD Layer Guidelines, Second Edition (with minor amendments) as a constituent document of the NCS Version 1.0, published in July 1999. The NCS Project Committee immediately set to work on publication of Version 2.0, which was published in 2002.

Considerable confusion resulted from the lack of "alignment" between the "Second Edition" of CAD Layer Guidelines and "Version 1.0" of the NCS. Because CAD Layer Guidelines, Second Edition was published before, and later incorporated into, the NCS Version 1.0, this could not be avoided. With publication of the NCS Version 2.0, this problem was corrected by giving the constituent document an entirely new name. For the first time, "AIA" became part of the title of the publication, and the numbered "editions" were abandoned. As a result, this publication became known as AIA CAD Layer Guidelines: U.S. National CAD Standard - Version 2.0. Subsequent editions of the NCS adopted the same nomenclature.

# Layer Name Format

## HIERARCHY OF DATA FIELDS

The layer name format is organized as a hierarchy. This arrangement allows users to select from a number of options for naming layers according to the level of detailed information desired. Layer names consist of distinct data fields separated from one another by dashes. A detailed list of abbreviations, or field codes, is prescribed to define the content of layers. Most field codes are mnemonic English abbreviations of construction terminology that are easy to remember.

There are four defined layer name data fields: Discipline Designator, Major Group, two Minor Groups, and Status. The Discipline Designator and Major Group fields are mandatory. The Minor Group and Status fields are optional. Each data field is separated from adjacent fields by a dash ("-") for clarity.

The complete NCS layer name format, showing the Discipline Designator, the Major Group, two Minor Groups, and the Status fields.

| A | I | - | W | A | L | L | - | F | U | L | L | - | D | I | M | S | - | N |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

## BEFORE YOU BEGIN

The NCS allows you to select from a number of format options for creating layer names. It is recommended that you select the options that you wish to use for layer names on a given project, and then apply the resulting format consistently for all layer names on that project.

NOTE: For conceptual conformance to ISO 13567, Organization and Naming of Layers for CAD, the layer name format and length must be the same for all layers on a given project. See CLG Appendix C - Complying with NCS and ISO 13567, CLG section 6.0 for information about ISO conformance. Δ

## DISCIPLINE DESIGNATOR, LEVEL 1

The Discipline Designator denotes the category of subject matter contained on the specified layer. The Discipline Designator is a two- character field. The first character is the discipline character, and the second character is an optional modifier. The Discipline Designator is described in greater detail in UDS Section 1.3. For a complete list of Discipline Designators see CLG Appendix A - Discipline Designators, CLG section 4.1 and UDS Appendix A - Discipline Designators, UDS section 1.6.

A typical layer name showing the required data fields only.

Note that only the mandatory discipline character is shown, creating a Level 1 Discipline Designator.

| A | - | W | A | L | L |
| --- | --- | --- | --- | --- | --- |

| LEVEL 1 DISCIPLINE DESIGNATORS |  |
| --- | --- |
| A | Architectural |
| B | Geotechnical |
| C | Civil |
| D | Process |
| E | Electrical |
| F | Fire Protection |
| G | General |
| H | Hazardous Materials |
| I | Interiors |
| L | Landscape |
| M | Mechanical |
| O | Operations |
| P | Plumbing |
| Q | Equipment |
| R | Resource |
| S | Structural |
| T | Telecommunications |
| V | Survey / Mapping |
| W | Distributed Energy |
| X | Other Disciplines |
| Z | Contractor / Shop Drawings |

## DISCIPLINE DESIGNATOR, LEVEL 2

A typical layer name showing the required data fields only.

Note that the mandatory Level 1 discipline character is supplemented by the optional discipline modifier to create a Level 2 Discipline Designator.

| A | D | - | W | A | L | L |
| --- | --- | --- | --- | --- | --- | --- |

The optional second character is used to further define the discipline character. As an example, the Level 2 Discipline Designators for Architectural are shown:

#### Designator	Description

A	Architectural

AD	Architectural Demolition

AE	Architectural Elements

AF	Architectural Finishes

AG	Architectural Graphics

AI	Architectural Interiors

AS	Architectural Site

AJ	User Defined

AK	User Defined

For a complete list of Discipline Designators see CLG Appendix A - List of Discipline Designators, Major and Minor Groups, and Status Fields, CLG section 4.1 and UDS Appendix A - Discipline Designators, UDS section 1.6.

A typical layer name showing the required data fields only. The mandatory Major Group field is highlighted:

MAJOR GROUP

| A | - | W | A | L | L |
| --- | --- | --- | --- | --- | --- |

The major group is a four-character field that identifies a major building system. The prescribed Major Group field codes (four- character abbreviations) shown on the Layer List are logically grouped with specific discipline designators. However, any Major Group may be combined with any prescribed Discipline Designator, provided that the definition of the Major Group remains unchanged.

Therefore, any reasonable combination of the prescribed Discipline Designators and Major Groups is permitted.

NOTE: The NCS recognizes that there will be instances where user-defined Major Group field codes will be required. The NCS set of Major Group field codes is not intended to be all inclusive. There will be instances when project specific Major Groups will need to be created. In these cases Major Group field codes are allowed, however, they must contain four alphabetic and/or numeric characters and/or "~", and must be fully documented on the NCS Compliance Disclosure Statement for the project or identified as project specific in the standard supplement in which they are used.

NOTE: For conceptual conformance to ISO 13567, Organization and Naming of Layers for CAD, the use of the Major Group "ANNO" is not permitted. See CLG Appendix C - Complying with NCS and ISO 13567, CLG section 6.0 for information about ISO conformance. Δ

A typical layer name showing one optional Minor Group field:

A typical layer name showing two optional Minor Group fields:

MINOR GROUP

| A | - | W | A | L | L | - | F | U | L | L |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

This is an optional, four-character field to further define the Major Groups. For example, A-WALL-FULL denotes Architectural, Wall, Full-height. A second minor group may be used for still further delineation of the data contained on a layer. For example, A-WALL- FULL TEXT indicates Architectural, Wall, Full-height, Text.

| A | - | W | A | L | L | - | F | U | L | L | - | T | E | X | T |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

The prescribed Minor Group field codes (four-character abbreviations) shown on the Layer List are logically grouped with specific Major Groups. However, any Minor Group may be used to modify any Major Group, provided that the definition of the Minor Group remains unchanged. Therefore, any reasonable combination of the prescribed Major and Minor Groups is permitted.

NOTE: User-defined Minor Group field codes are permitted. They must contain four alphabetic and/or numeric characters and/or "~", and must be fully documented on the NCS Compliance Disclosure Statement for the project on which they are used.

NOTE: For conceptual conformance to ISO 13567, Organization and Naming of Layers for CAD, the use of certain Minor Group field codes is restricted. See CLG Appendix C - Complying with NCS and ISO 13567, CLG section 6.0 for information about ISO conformance. Δ

A typical layer name showing the location of the optional Status field:

STATUS (PHASE)

| A | - | W | A | L | L | - | F | U | L | L | - | T | E | X | T | - | N |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

The status field is an optional single-character field that distinguishes the data contained on the layer according to the status of the work or the construction phase. The prescribed field codes for this field are as follows:

STATUS FIELD CODES

| A | Abandoned |
| --- | --- |
| D | Existing to demolish |
| E | Existing to remain |
| F | Future work |
| M | Items to be moved |
| N | New work |
| T | Temporary work |
| X | Not in contract |
| 1-9 | Phase numbers |

NOTE: For conceptual conformance to ISO 13567, Organization and Naming of Layers for CAD, this field may be used to denote either "Status" OR "Phase," but not BOTH. See CLG Appendix C - Complying with NCS and ISO 13567, CLG section 6.0 for information about ISO conformance. Δ

# Drawing View Layer List

## DRAWING VIEW FIELD CODES

The Drawing View field codes are specialized codes for layers that are organized primarily by drawing type, rather than by major building system. The field codes DETL, ELEV, and SECT may also be used as Minor Group field codes to modify a major building system.

For data sets that are organized by drawing type, an optional alphanumeric Minor Group field code, ANNN, is prescribed to further distinguish drawings within a single CAD file. This Minor Group may be used ONLY to modify the prescribed Drawing View Major Groups; it may not be used to modify any other Major Group. The format of ANNN is also prescribed. It must consist of a single alphabetic character followed by a three-digit number between 001 and 999. The definition of ANNN is not prescribed; it must be defined by the user. The definition must be documented on the NCS Compliance Disclosure Statement for the project on which it is used.

The Minor Group field codes IDEN, MBND, MCUT, OTLN, and PATT may be used to modify any Major or Minor Group in the Layer List. The definitions of these prescribed field codes cannot be changed. See CLG Sections 1.5 and 1.6 for rules and options governing the use of field codes.

## DRAWING VIEW LAYER NAMES

#### Layer Name	Description

□□-DETL	Detail

□□-ELEV	Elevation

□□-SECT	Section

□□-□□□□-ANNN	Drawing View Major Group: optional number (A = letter, NNN = number between 001 and 999)

□□-□□□□-ANNN-IDEN  Drawing View Major Group: optional number: identification tags

□□-□□□□-ANNN-MBND Drawing View Major Group: optional number: material beyond cut

□□-□□□□-ANNN-MCUT Drawing View Major Group: optional number: material cut by the view

□□-□□□□-ANNN-OTLN Drawing View Major Group: optional number: outline

□□-□□□□-ANNN-PATT Drawing View Major Group: optional number: textures and hatch patterns

# Annotation Layer List

## ANNOTATION FIELD CODES

Annotation consists of text, dimensions, notes, sheet borders, detail references and other elements on CAD drawings that do not represent physical aspects of a building. Use of the Major Group ANNO allows all annotation to be placed in a defined group of layers.

The Layer Names shown below provide examples for the use of Minor Group field codes for annotation. These Minor Groups may be used to modify any Major or Minor Group in the Layer List. See CLG Sections 1.5 and

1.6 for complete rules and options governing the use of Major and Minor Group field codes.

## ANNOTATION LAYER NAMES

#### Layer Name	Description

□□-ANNO	Annotation

□□-□□□□-BRNG	Bearings and distance labels (survey coordinates)

□□-□□□□-DIMS	Dimensions

□□-□□□□-IDEN	Identification tags

□□-□□□□-KEYN	Keynotes

□□-□□□□-LABL	Labels

□□-□□□□-LEGN	Legends, symbol keys

□□-□□□□-LOGO	Company logo

□□-□□□□-MARK	Markers, break marks, leaders

□□-□□□□-MATC	Match lines

□□-□□□□-NOTE	Notes

□□-□□□□-NPLT	Non-plotting graphic information

□□-□□□□-PROS	Date/Time/File name stamp

□□-□□□□-RDME	Read-me layer (not plotted)

□□-□□□□-REDL	Redlines

□□-□□□□-REFR	Reference, external files

□□-□□□□-REVC	Revision clouds

□□-□□□□-REVS	Revision indicators and text

□□-□□□□-SCHD	Schedules

□□-□□□□-STMP	Professional stamps

□□-□□□□-SYMB	Reference symbols

□□-□□□□-TABL	Data tables

□□-□□□□-TEXT	Text

□□-□□□□-TITL	Drawing or detail titles

□□-□□□□-TTLB	Border and title block

# Appendix A - List of Discipline Designators, Major and Minor Groups, and Status Fields

The CLG states that "any Major Group may be combined with any prescribed Discipline Designator, provided that the definition of the Major Group remains unchanged" and "any Minor Group may be used to modify any Major Group, provided that the definition of the Minor Group remains unchanged." Therefore the following alphabetical list of all Discipline Designators, Major and Minor Groups, and Status Fields regardless of discipline has been compiled for easy reference.

    - 4.1 Discipline Designators

    - 4.2 Major Groups

    - 4.3 Minor Groups

    - 4.4 Status Fields

## DISCIPLINE DESIGNATORS

DOWNLOAD SPREADSHEET

#### Designator	Description

A	Architectural

AD	Architectural Demolition

AE	Architectural Elements

AF	Architectural Finishes

AG	Architectural Graphics

AI	Architectural Interiors

AJ	User Defined

AK	User Defined

AS	Architectural Site

B	Geotechnical

BJ	User Defined

BK	User Defined

C	Civil

CD	Civil Demolition

CG	Civil Grading

CI	Civil Improvements

CJ	User Defined

CK	User Defined

CN	Civil Nodes

CP	Civil Paving

CS	Civil Site

CT	Civil Transportation

CU	Civil Utilities

D	Process

DA	Process Airs

DC	Process Chemicals

DD	Process Demolition

DE	Process Electrical

DG	Process Gases

DI	Process Instrumentation

| DJ | User Defined |
| --- | --- |
| DK | User Defined |
| DL | Process Liquids |
| DM | Process HPM Gases |
| DO | Process Oils |
| DP | Process Piping |
| DQ | Process Equipment |
| DR | Process Drains and Reclaims |
| DS | Process Site |
| DV | Process Vacuum |
| DW | Process Waters |
| DX | Process Exhaust |
| DY | Process Slurry |
| E | Electrical |
| ED | Electrical Demolition |
| EI | Electrical Instrumentation |
| EJ | User Defined |
| EK | User Defined |
| EL | Electrical Lighting |
| EP | Electrical Power |
| ES | Electrical Site |
| ET | Electrical Telecommunications |
| EY | Electrical Auxiliary Systems |
| F | Fire Protection |
| FA | Fire Detection and Alarm |
| FJ | User Defined |
| FK | User Defined |
| FX | Fire Suppression |
| G | General |
| GC | General Contractual |
| GI | General Informational |
| GJ | User Defined |
| GK | User Defined |
| GR | General Resource |
| H | Hazardous Materials |
| HA | Hazardous Materials Asbestos |
| HC | Hazardous Materials Chemicals |
| HJ | User Defined |
| HK | User Defined |
| HL | Hazardous Materials Lead |

| HP | Hazardous Materials PCB |
| --- | --- |
| HR | Hazardous Materials Refrigerants |
| I | Interior |
| ID | Interior Demolition |
| IF | Interior Furnishings |
| IG | Interior Graphics |
| IJ | User Defined |
| IK | User Defined |
| IN | Interior Design |
| L | Landscape |
| LD | Landscape Demolition |
| LG | Landscape Grading |
| LI | Landscape Irrigation |
| LJ | User Defined |
| LK | User Defined |
| LL | Landscape Lighting |
| LP | Landscape Planting |
| LR | Landscape Relocation |
| LS | Landscape Site |
| M | Mechanical |
| MD | Mechanical Demolition |
| MH | Mechanical HVAC |
| MI | Mechanical Instrumentation |
| MJ | User Defined |
| MK | User Defined |
| MP | Mechanical Piping |
| MS | Mechanical Site |
| O | Operations |
| OJ | User Defined |
| OK | User Defined |
| P | Plumbing |
| PD | Plumbing Demolition |
| PJ | User Defined |
| PK | User Defined |
| PL | Plumbing |
| PP | Plumbing Piping |
| PQ | Plumbing Equipment |
| PS | Plumbing Site |
| Q | Equipment |
| QA | Equipment Athletic |

| QB | Equipment Bank |
| --- | --- |
| QC | Equipment Dry Cleaning |
| QD | Equipment Detention |
| QE | Equipment Educational |
| QF | Equipment Food service |
| QH | Equipment Hospital |
| QJ | User Defined |
| QK | User Defined |
| QL | Equipment Laboratory |
| QM | Equipment Maintenance |
| QP | Equipment Parking Lot |
| QR | Equipment Retail |
| QS | Equipment Site |
| QT | Equipment Theatrical |
| QV | Equipment Video / Photographic |
| QY | Equipment Security |
| R | Resource |
| RA | Resource Architectural |
| RC | Resource Civil |
| RE | Resource Electrical |
| RJ | User Defined |
| RK | User Defined |
| RM | Resource Mechanical |
| RR | Resource Real Estate |
| RS | Resource Structural |
| S | Structural |
| SB | Structural Substructure |
| SD | Structural Demolition |
| SF | Structural Framing |
| SJ | User Defined |
| SK | User Defined |
| SS | Structural Site |
| T | Telecommunications |
| TA | Telecommunications Audio Visual |
| TC | Telecommunications Clock and Program |
| TI | Telecommunications Intercom |
| TJ | User Defined |
| TK | User Defined |
| TM | Telecommunications Monitoring |
| TN | Telecommunications Data Networks |

TT	Telecommunications Telephone

TY	Telecommunications Security

V	Survey / Mapping

VA	Survey / Mapping Aerial

VC	Survey / Mapping Computated Points

VF	Survey / Mapping Field

VI	Survey / Mapping Digital

VJ	User Defined

VK	User Defined

VN	Survey / Mapping Node Points

VS	Survey / Mapping Staked Points

VU	Survey / Mapping Combined Utilities

W	Distributed Energy

WC	Distributed Energy Civil

WD	Distributed Energy Demolition

WI	Distributed Energy Interconnection

WJ	User Defined

WK	User Defined

WP	Distributed Energy Power

WS	Distributed Energy Structural

WT	Distributed Energy Telecommunications

WY	Distributed Energy Auxiliary Systems

X	Other Disciplines

XJ	User Defined

XK	User Defined

Z	Contractor/Shop Drawings

ZJ	User Defined

ZK	User Defined

## MAJOR GROUPS

#### Major Group Layer Name  Description

ACCS	Access

ACID	Acid waste systems

AERI	Aerial Survey

AFFF	Aqueous film-forming foam system

AFLD	Airfields

AIR~	Air

ALGN	Alignment

ALRM	Alarm system

| ANNO | Annotation |
| --- | --- |
| AREA | Area |
| AUXL | Auxiliary systems |
| BARR | Barrier |
| BCST | Broadcast related system (radio or TV) |
| BEAM | Beams |
| BELL | Bell system |
| BLDG | Buildings and primary structures |
| BLIN | Baseline |
| BNDY | Political boundaries |
| BORE | Borings |
| BRCG | Bracing |
| BRDG | Bridge |
| BRIN | Brine systems |
| BRKL | Break / fault lines |
| BRLN | Building restriction line |
| BZNA | Buffer zone area |
| CABL | Cable systems |
| CATH | Cathodic Protection System |
| CATV | Cable television system |
| CCTV | Closed-circuit television system |
| CEME | Cemetery |
| CHAN | Navigable channels |
| CHEM | Chemical |
| CHIM | Chimneys and stacks |
| CLNG | Ceiling |
| CLOK | Clock system |
| CMPA | Compressed / processed air systems |
| CMPR | Computer |
| CNDW | Condenser water systems |
| CO2S | CO2 system |
| CODE | Code compliance plan |
| COLS | Columns |
| COMM | Communications |
| CONT | Controls and instrumentation |
| CONV | Conveying systems |
| CRPT | Carpet / carpet tiles |
| CSWK | Casework |
| CTRL | Control points |
| CWTR | Chilled water systems |

| DATA | Data / LAN system |
| --- | --- |
| DECK | Deck |
| DETL | Detail |
| DFLD | Drain fields |
| DIAG | Diagrams |
| DICT | Dictation system |
| DOMW | Domestic water systems |
| DOOR | Doors |
| DRAN | Drains |
| DRIV | Driveways |
| DTCH | Ditches or washes |
| DUAL | Dual temperature systems |
| DUST | Dust and fume collection systems |
| ELEC | Electrical system, telecom plan |
| ELEV | Elevation |
| ELHT | Electric heat |
| EMCS | Energy monitoring control system |
| ENER | Energy management systems |
| EQPM | Equipment |
| EROS | Erosion and sediment control |
| ESMT | Easements |
| EVAC | Evacuation plan |
| EXHS | Exhaust system |
| FENC | Fences |
| FIRE | Fire protection |
| FLHA | Flood hazard area |
| FLOR | Floor |
| FNDN | Foundation |
| FNSH | Finishes |
| FRAM | Braced frame or moment frame |
| FSTN | Fasteners and connections |
| FUEL | Fuel systems |
| FUME | Fume hood |
| FURN | Furnishings |
| GAS~ | Gas |
| GATE | Gate |
| GLAZ | Glazing |
| GLYC | Glycol systems |
| GRID | Grids |
| GRLN | Grade line |

| GRND | Ground system |
| --- | --- |
| HALN | Halon |
| HWTR | Hot water heating system |
| HVAC | HVAC systems |
| HYDR | Hydraulic structure |
| IGAS | Inert gas |
| INGR | Ingrants |
| INST | Instrumentation system |
| INTC | Intercom / PA systems |
| IRRG | Irrigation |
| JNTS | Joints |
| JOIS | Joists |
| LAND | Land |
| LEGN | Legend, symbols keys |
| LEVE | Levee |
| LGAS | Laboratory gas systems |
| LIQD | Liquid |
| LITE | Lighting |
| LNTL | Lintels |
| LOCN | Limits of construction |
| LTNG | Lightning protection system |
| MACH | Machine shop |
| MAJQ | Major equipment |
| MDGS | Medical gas systems |
| MILL | Millwork |
| MINQ | Minor equipment |
| MKUP | Make-up air systems |
| MNTG | Mounting system |
| MPIP | Miscellaneous piping systems |
| NGAS | Natural gas systems |
| NODE | Node |
| NURS | Nurse call system |
| OBST | Obstructions |
| OIL~ | Oil |
| OTGR | Outgrants |
| PADS | Pads |
| PERC | Perc testing |
| PGNG | Paging system |
| PHON | Telephone system |
| PIPE | Piping |

| PLAN | Key Plan (Floor Plan) |
| --- | --- |
| PLAT | Platform |
| PLNT | Plant and landscape material |
| POND | Ponds |
| POWR | Power |
| PRKG | Parking lots |
| PROC | Process systems |
| PROJ | Projector system |
| PROP | Property |
| PROT | Fire protection system |
| PRTN | Partitions |
| PVMD | Photovoltaic modules |
| PVMT | Pavement |
| RAIL | Railroad |
| RAIR | Relief air systems |
| RCOV | Energy recovery systems |
| REFG | Refrigeration systems |
| RIGG | Rigging / automation systems |
| RIVR | River |
| ROAD | Roadways |
| ROOF | Roof |
| RRAP | Riprap |
| RUNW | Runway |
| RWAY | Right-of-way |
| SECT | Section |
| SERT | Security system |
| SGHT | Sight distance |
| SIGN | Sign |
| SITE | Site features |
| SLAB | Slab |
| SLUR | Slurry |
| SMOK | Smoke extraction systems |
| SOIL | Soils |
| SOUN | Sound system |
| SPCL | Special systems |
| SPFX | Entertainment special effects system |
| SPKL | Sprinkler |
| SSWR | Sanitary sewer |
| STEM | Steam system |
| STIF | Stiffener |

STRM	Storm sewer

STRS	Stairs

SURV	Survey

SWLK	Sidewalks

TEST	Test equipment

TILE	Tile

TINN	Triangulated irregular network

TOPO	Topographic feature

TRAL	Trails or paths

TRAN	Transmission system

TRUS	Trusses

TVAN	Television antenna system

TVVS	Television and video system

UNID	Unidentified site objects

UTIL	Utilities

VACU	Vacuum

VIDO	Entertainment projection systems

WALL	Walls

WATR	Water supply

WETL	Wetlands

WIND	Wind powered

WWAY	Waterway

## MINOR GROUPS

#### Minor Group Layer Name  Description

025Y	25-year mark

04FT	Four feet high

050Y	50-year mark

06FT	Six feet high

100Y	100-year mark

200Y	200-year mark

AA~~	Agitation air-system

ABLT	Anchor bolts

ABOV	Above

ABUT	Abutment

ACCS	Access

ACFU	Fused ac

ACTL	Aerial horizontal and vertical control points

ACNF	Unfused ac

AGGR	Exposed aggregate

AIR~	Air

ALOC	Allocation

ALRM	Alarm

ALUM	Aluminum

AMEX	Ammonia exhaust-system

AMW~	Ammonia waste-system

ANNN	Optional number

(A = letter, NNN = number between 001 and 999)

ANNO	Annotation

ANOD	Sacrificial anode

AR~~	Argon-system

ARB~	Argon bulk-system

ARC~	Regenerative caustic-system

AREX	Arsenic exhaust-system

ASPH	Asphalt

BA~~	Breathable air-system

BACK	Back

BAFL	Baffle block and splash pad

BARR	Barrier

BASN	Stilling and settling basin

BBAC	Battery backup

BEDS	Perennial and annual beds

BENT	Top of bent

BFW~	Boiler feed water-system

BKRS	Breakers

BLBD	Boiler blow down piping

BLDG	Building points

BLIN	Baseline

BMRK	Benchmarks

BNDY	Boundary

BOLD	Bold lines

BORO	Borough

BOT1	Bottom group 1

BOT2	Bottom group 2

BOTB	Bottom of bank

BOTM	Bottom

BOXD	Mixing box, dual duct

BOXS	Mixing box, single duct

BRCK	Brick

BRDG	Bridge

| BRGX | Bridging |
| --- | --- |
| BRKL | Break lines |
| BRNG | Bearings and distance labels |
| BROW | Brush row points |
| BRSH | Brush points |
| BUOY | Buoy |
| BUSH | Bushes and shrubs |
| BUSS | Bus duct |
| BUSW | Busways |
| BUT~ | Butane-system |
| BWTR | Breakwater |
| C~~~ | Caustic-system |
| CA~~ | Compressed air-system |
| CABL | Cable |
| CAIR | Compressed air |
| CARS | Cars and other vehicles |
| CATV | Cable television |
| CAVI | Cavity |
| CBOX | Combiner box |
| CD~~ | Condensate drain-system |
| CDA~ | Clean dry air-system |
| CDFF | Ceiling diffusers |
| CHIM | Chimney |
| CIPR | Culvert inlet protection |
| CIRC | Circuits |
| CITY | City |
| CLAS | Classifications |
| CLDA | Cold air |
| CLG~ | Chlorine gas-system |
| CLHD | Ceiling heads |
| CLNG | Ceiling |
| CLV~ | Chlorine vacuum-system |
| CLW~ | Concentrated lead waste-system |
| CMTL | Corrugated metal |
| CMUW | Concrete masonry unit |
| CMW~ | Concentrated metals waste-system |
| CNDS | Condensate piping |
| CNDT | Diversionary/bypass conduit/culvert |
| CNMB | Circuit numbers |
| CNTE | Construction entrance |

| CNTJ | Construction joint |
| --- | --- |
| CNTR | Center |
| CNTY | County |
| COAX | Coax cable |
| COFF | Coffer dam |
| CONC | Concrete |
| CONI | Coniferous trees |
| CONS | Conservation |
| CORP | Corporation |
| COVR | Coverage |
| CPIP | Cold water piping |
| CRIT | Critical |
| CRKT | Crickets |
| CSTG | Construction/Grading |
| CSWK | Casework |
| CTLA | Controlled access |
| CTLJ | Control joint |
| CTNR | Container or planter |
| CUPW | Copper plating waste-system |
| CURB | Curb |
| CURR | Impress current |
| CURT | Curtain |
| CURV | Curve |
| CURW | Copper rinse waste-system |
| CUSW | Copper slurry waste-system |
| CV~~ | Chemical vacuum-system |
| DACL | De-Authorized channel limits, anchorages, etc. |
| DAM~ | Dam |
| DASP | Description attributes for survey points |
| DATA | Data |
| DCFU | Fused dc |
| DCNF | Unfused dc |
| DDIV | Drainage divides |
| DECK | Deck |
| DEPR | Depression |
| DEV~ | Developer-system |
| DEVC | Devices |
| DFEE | Disposed fee |
| DIAG | Diagrams |
| DIMS | Dimensions |

| DIR~ | De-Ionized water return-system |
| --- | --- |
| DIRC | DI reclaim-system |
| DIS~ | De-Ionized water supply-system |
| DISC | Discharge |
| DIWP | DI polishing loop-system |
| DLPH | Dolphin |
| DLW~ | Dilute waste-system |
| DMPR | Fire, smoke, volume damper |
| DOCK | Decks, docks, floats, piers |
| DOOR | Equipment doors |
| DRAN | Drainage slope indications |
| DRIP | Drip irrigation tubing |
| DRIV | Driveway points |
| DRNS | Drains |
| DSCO | Disconnect switches |
| DTCH | Ditches or washes |
| DUCT | Ductwork |
| DVDK | Diversion dike |
| DVDR | Thin dividers |
| EASP | Elevation attributes for survey points |
| EDGE | Edge |
| EDGR | Planting bed edger |
| EFAN | Equipment with electric fans |
| EG~~ | Ethylene glycol-system |
| EGW~ | Ethylene glycol waste-system |
| ELEC | Electrical |
| ELEV | Elevation |
| EMER | Emergency |
| ENCL | Equipment enclosures |
| ENGR | Engineering Information |
| EPDU | Equipment with piping, ductwork and electricity |
| EPIP | Equipment with piping and electricity |
| EQPM | Equipment |
| EQUI | Equipotential |
| ERTH | Earth |
| ESMT | Easement |
| EV~~ | Equipment vacuum-system |
| EVGR | Evergreen trees-broadleaf |
| EVTR | Elevator cars and equipment |
| EWAT | Edge of water |

| EXHS | Exhaust air |
| --- | --- |
| EXIT | Exit |
| EXPJ | Expansion joint |
| EXTI | Extinguishers |
| EXTR | Exterior |
| FACE | Face |
| FALT | Fault/break lines |
| FDPL | Flood plain |
| FDTA | Field data |
| FEE~ | Fee |
| FEED | Feeders |
| FENC | Fences |
| FEND | Fender |
| FIBR | Fiber optics cable |
| FILE | File cabinets |
| FILL | Fill and cover material |
| FINE | Fine lines |
| FIRE | Fire protection |
| FISH | Fish ladder/passage |
| FIXD | Fixed |
| FIXT | Fixtures |
| FLDR | Floor drains |
| FLLW | Flow |
| FLNE | Fire lane |
| FLOR | Floor |
| FLOW | Flowline |
| FLPL | Flagpole |
| FLUM | Flume |
| FLYS | Fly station |
| FNSH | Finishes |
| FORC | Force main |
| FREE | Freestanding |
| FRMG | Framing |
| FTNG | Footings |
| FTPT | Area footprints |
| FULL | Full-height |
| FURN | Furnishings |
| FW~~ | Fire water-system |
| GAGE | Gauge |
| GCVR | Ground cover |

| GENF | General features |
| --- | --- |
| GGEP | Gas general piping |
| GLAZ | Glazing |
| GNDW | Ground water |
| GPRP | Gas process piping |
| GRAL | Guard rail |
| GRBM | Grade beams |
| GRID | Grid |
| GRIL | Grilles |
| GRND | Ground |
| GRTG | Grating |
| GRVL | Gravel |
| H2~~ | Hydrogen-system |
| H2O2 | Hydrogen peroxide-system |
| HCDA | High pressure clean dry air-system |
| HCL~ | Hydrochloric acid-system |
| HDIR | Hot DI return-system |
| HDIS | Hot DI supply-system |
| HDLN | Hidden line |
| HDRC | Hot DI reclaim-system |
| HE~~ | Helium-system |
| HEAD | Door and window headers |
| HF~~ | Hydrofluoric acid-system |
| HFW~ | Hydrofluoric waste-system |
| HIDD | Objects or lines hidden from view |
| HOLE | Holes |
| HORZ | Horizontal |
| HOSE | Hoses |
| HOTA | Hot air |
| HPDR | High pH DI return-system |
| HPDS | High pH DI supply-system |
| HPIP | Hot water/high-pressure piping |
| HPN2 | High purity nitrogen-system |
| HPO2 | High purity oxygen-system |
| HRAL | Handrails/guard rails |
| HRDW | Hardware |
| HSSS | Hollow structural steel |
| HTCH | Hatch |
| HTEX | Heat exhaust-system |
| HV~~ | House vacuum-system |

| HVA~ | Arsenic house vacuum-system |
| --- | --- |
| HVAC | HVAC systems |
| HVPT | Horizontal/vertical |
| HWAL | Headwall |
| HYDT | Hydrants and connections |
| IA~~ | Instrument air-system |
| ICW~ | Industrial city water-system |
| IDEN | Identification tags |
| INEG | Ingress/egress |
| INPR | Inlet protection |
| INST | Instrumentation |
| INTK | Intake |
| INTR | Interior |
| IPA~ | Isopropyl alcohol-system |
| IW~~ | Industrial waste-system |
| JACK | Jacks |
| JAMB | Door and window jambs |
| JBOX | Junction box |
| JNTC | Control joint |
| JNTE | Expansion joint |
| KEYN | Keynotes |
| LABL | Labels |
| LADD | Ladders and ladder assemblies |
| LATL | Lateral line |
| LCHE | Leak check helium-system |
| LDTA | Laboratory data |
| LEAS | Lease |
| LEGN | Legend, symbol keys |
| LEVL | Level changes |
| LFEE | Disposed less than fee |
| LICN | License |
| LIMI | Limit of earthwork |
| LINE | Lines |
| LINK | Chain link |
| LMTA | Limited access |
| LO~~ | Lube oil-system |
| LOGO | Company logo |
| LONG | Longitudinal |
| LOWR | Lower |
| LPG~ | Liquid petroleum gas-system |

| LPIP | Low-pressure piping |
| --- | --- |
| LQPG | Liquid petroleum gas |
| LSCP | Landscape |
| LTRL | Lateral pipe |
| MAIN | Mainline |
| MAJR | Major |
| MARK | Markers, break marks, leaders |
| MATC | Match lines |
| MBND | Material beyond cut |
| MCUT | Material cut by the view |
| MEDM | Medium lines |
| MESH | Mesh or wire |
| METL | Metal |
| MHOL | Manhole |
| MINR | Minor |
| MISC | Miscellaneous |
| MKUP | Make-up water |
| MLCH | Mulches-organic and inorganic |
| MNTG | Mounting system |
| MOOR | Mooring |
| MOVE | Movable |
| MPIP | Medium-pressure piping |
| MRKG | Pavement markings |
| MRKR | Marker |
| MSNW | Masonry |
| MULT | Multi-conductor cable |
| MVNG | Moving/Suspended |
| MW~~ | Metals waste-system |
| N2~~ | Nitrogen-system |
| N2O~ | Nitrous oxide-system |
| NAID | Navigation aids |
| NATL | National |
| NFEE | Non-fee |
| NG~~ | Natural gas-system |
| NGAS | Natural gas line |
| NITG | Nitrogen |
| NOTE | Notes |
| NOVR | Non-overflow structure |
| NOXG | Nitrous oxide |
| NPLT | Non-plotting graphic information |

| NPW~ | Non-potable water-system |
| --- | --- |
| NPWR | Non-potable water reuse-system |
| NSBR | Noise barrier |
| O2~~ | Oxygen-system |
| OA~~ | Outside air-system |
| OBJT | Objects |
| OCCP | Occupant or employee names |
| ODFF | Other diffusers |
| OFA~ | Oil-free air-system |
| OFST | Offset zones |
| OGEP | Oil general piping |
| OIW~ | Organic industrial waste-system |
| OLW~ | Organic liquid waste-system |
| OPNG | Openings |
| OPNX | Opening indication |
| OPRP | Oil process piping |
| OSW~ | Organic solvent waste-system |
| OTHD | Other heads |
| OTLN | Outline |
| OVHD | Overhead |
| OXYG | Pure O2 |
| PA~~ | Plant air-system |
| PADM | Pad-mounted |
| PADS | Pads |
| PALM | Palm trees |
| PANL | Panels |
| PASP | Point number attributes for survey points |
| PATT | Texture or hatch patterns |
| PAVR | Unit pavers |
| PCAP | Pile caps |
| PCST | Pre-cast concrete |
| PCWR | Cooling water return-system |
| PCWS | Cooling water supply-system |
| PENE | Penetrations |
| PENS | Penstock |
| PEQP | Process equipment |
| PERI | Perimeter |
| PERM | Permanent |
| PHON | Telephone line |
| PHOS | Phosphoric acid-system |

| PHRC | Phosphoric acid reclaim-system |
| --- | --- |
| PIER | Drilled piers |
| PILE | Piles |
| PIPE | Piping |
| PLAY | Play structures |
| PLNT | Plants |
| PLYW | Plywood |
| PMIT | Permit |
| PNHS | Penthouse |
| PNLS | System panels |
| PNPT | Panel points |
| POCC | Point of common coupling |
| POI~ | Point of interconnection |
| POLE | Poles |
| POLM | Pole-mounted |
| POND | Retention pond |
| POOL | Pools and spas |
| POST | Posts |
| PPIP | Process piping |
| PRCH | Porch |
| PRCL | Parcels |
| PRHT | Partial-height |
| PRIM | Primary |
| PRKG | Parking |
| PRO~ | Propane-system |
| PROF | Profile |
| PROS | Date/time/file name stamp |
| PROV | Province |
| PRPT | Parapet |
| PRVC | Privacy |
| PSW~ | Photo solvent waste-system |
| PV~~ | Vacuum-system |
| PVMT | Pavement |
| PW~~ | Potable water-system |
| QTRS | Quarter section |
| RAIS | Raised |
| RAMP | Accessible ramp |
| RATE | Ratings |
| RBAR | Reinforcing bar |
| RCON | Reinforced concrete |

| RDFF | Return air diffusers |
| --- | --- |
| RDGE | Roof ridges |
| RDME | Read-me layer (not plotted) |
| REDL | Redlines |
| REFR | Reference, external files |
| RER~ | Solvent-system |
| RETN | Return |
| REVC | Revision clouds |
| REVS | Revision indicators and text |
| RFDR | Roof drains |
| RFEQ | Rooftop equipment |
| RISR | Risers |
| RO~~ | Reverse osmosis water-system |
| ROAD | Roadway |
| ROCK | Large rocks and rock outcroppings |
| ROOF | Roof |
| ROR~ | Reverse osmosis reject water-system |
| RPIP | Recirculation piping |
| RRAP | Riprap |
| RSCH | Sketch line round or oval duct |
| RSRV | Reservation |
| RTWL | Retaining wall |
| RWAY | Right-of-way |
| SAIR | Scavenge air |
| SATD | Satillite dishes |
| SAUD | Audio signal |
| SBCK | Setback lines |
| SBST | Substations |
| SCEX | Scrubber exhaust-system |
| SCHD | Schedules |
| SCOM | Communications signal |
| SCTL | Control signal |
| SDAT | Data signal |
| SDD~ | Scrubber duct drains-system |
| SDFF | Supply diffusers |
| SDGA | Digital audio signal |
| SDGV | Digital video signal |
| SEAT | Seating |
| SECD | Secondary |
| SECT | Section |

| SEED | Seeding areas |
| --- | --- |
| SG~~ | Specialty gas-system |
| SGHT | Sight distance |
| SHAD | Shadow area |
| SHEA | Structural bearing or shear walls |
| SHLF | Wall-mounted shelving |
| SIGN | Signage |
| SILL | Window sills |
| SILT | Silt fence |
| SIZE | Ductwork size |
| SKCH | Sketch |
| SKLT | Skylight |
| SLR~ | Slurry return-system |
| SLS~ | Slurry supply-system |
| SLVE | Pipe sleeve |
| SLW~ | Slurry waste-system |
| SMIC | Microphone signal |
| SMOK | Smoke detector/heat sensors |
| SOUN | Soundings |
| SPCL | Special/specialties |
| SPKL | Sprinklers |
| SPLY | Supply |
| SPOT | Spot elevations |
| SPRT | Sports fields |
| SPWR | Power signal |
| SRFI | RF signal |
| SRGB | RGB and component video signal |
| SSCH | Sketch line rectangular duct |
| SSLT | Super silt fence |
| SSWR | Sanitary sewer |
| SSYN | Sync signal |
| STAN | Stationing |
| STAT | State |
| STBY | Standby |
| STEL | Steel |
| STEP | Steps |
| STMP | Professional stamp |
| STOR | Storage |
| STRC | Structures |
| STRM | Storm Sewer |

| STRP | Striping |
| --- | --- |
| STRS | Stair treads |
| SUBA | Cabinet sub-assemblies, drawer boxes |
| SUBD | Subdivision (interior) lines |
| SUBS | Sub-surface areas |
| SULF | Sulfuric acid-system |
| SULR | Sulfuric acid reclaim-system |
| SUPT | Support |
| SURF | Surface areas |
| SUSP | Suspended elements |
| SVEX | Solvent exhaust-system |
| SVID | Video signal |
| SW~~ | Solvent waste-system |
| SWAY | Spillway |
| SWBD | Switchboards |
| SWCH | Switches |
| SWF~ | Solvent waste flammable-system |
| SWLK | Sidewalks |
| SWMT | Storm water management |
| SWNF | Solvent waste non-flammable-system |
| SXTS | Sixteenth section |
| SYMB | Reference symbols |
| TABL | Data tables |
| TAKE | Taking lines |
| TANK | Storage tanks |
| TDIR | Tempered DI return-system |
| TDIS | Tempered DI supply-system |
| TEES | Main tees |
| TEMP | Temporary |
| TEST | Test stations |
| TEXT | Text |
| THER | Thermostats |
| TICK | Tick marks |
| TITL | Drawing or detail titles |
| TMAH | TMAH-system |
| TOP~ | Top |
| TOP1 | Top group 1 |
| TOP2 | Top group 2 |
| TOPB | Top of bank |
| TOWR | Towers |

| TPIT | Test pits |
| --- | --- |
| TPTN | Toilet partitions |
| TRAC | Tract lines |
| TRAK | Track |
| TRAL | Trail or path |
| TRAV | Transverse |
| TRAY | Cabletray and wireways |
| TREE | Trees |
| TROW | Tree row |
| TSHP | Town or township |
| TTLB | Border and titleblock |
| TURF | Lawn areas |
| TW~~ | Tempered water-system |
| UCPT | Under-carpet wiring |
| UCTR | Under counter |
| UN2~ | Utility nitrogen-system |
| UGND | Underground |
| UPPR | Upper |
| UPRW | Ultra-pure recycle water-system |
| UPS~ | Uninterruptible power supply |
| UPVD | Unpaved surface |
| UPW~ | Ultra-pure water-system |
| URAC | Under-floor raceways |
| UTIL | Utility lines |
| V~~~ | Vent-system |
| VACU | Vacuum |
| VALV | Valves |
| VEGE | Trees, shrubs, and other vegetation |
| VENR | Veneer |
| VENT | Vents |
| VERT | Vertical |
| VIEW | Triangulation view |
| VINE | Vines |
| VN2~ | Venturi nitrogen-system |
| VOID | Void regions |
| W2XS | Dimension lumber |
| WALL | Wall |
| WAR~ | Weld argon-system |
| WATR | Water supply |
| WDWK | Architectural woodwork |

WEIR	Pool weir

WELL	Well

WHIT	White paint

WIRE	Wiring

WKSF	Worksurface

WOOD	Wood

XFMR	Transformers

XTRU	Extrusion

YELO	Yellow paint

ZONE	Zoning

## STATUS FIELDS

#### Codes	Description

A	Abandoned

D	Existing to demolish

E	Existing to remain

F	Future work

M	Items to be moved

N	New work

T	Temporary work

X	Not in contract

1	Phase number 1

2	Phase number 2

3	Phase number 3

4	Phase number 4

5	Phase number 5

6	Phase number 6

7	Phase number 7

8	Phase number 8

9	Phase number 9

# Appendix B - Common Layer Lists by Discipline

The following lists of layers present the most commonly used layers for each discipline. The definitive list of Discipline Designators, Major and Minor Groups, and Status Fields is in CLG Appendix A - List of Discipline Designators, Major and Minor Groups, and Status Fields, CLG section 4.0.

    - 5.1 Architectural Layer List

    - 5.2 Civil Layer List

    - 5.3 Contractor/Shop Drawing Layer List

    - 5.4 Electrical Layer List

    - 5.12 Landscape Layer List

    - 5.13 Mechanical Layer List

    - 5.14 Operations Layer List

    - 5.15 Plumbing Layer List

    - 5.5 Distributed Energy Layer List

    - 5.6 Equipment Layer List

    - 5.7 Fire Protection Layer List

    - 5.8 General Layer List

    - 5.9 Geotechnical Layer List

    - 5.10 Hazardous Materials Layer List

    - 5.11 Interiors Layer List

    - 5.16 Process Layer List

    - 5.17 Resource Layer List

    - 5.18 Structural Layer List

    - 5.19 Survey/Mapping Layer List

    - 5.20 Telecommunications Layer List

    - 5.21 Other Disciplines Layer List

## ARCHITECTURAL LAYER LIST

### Architectural Field Codes

The Layer Names shown below provide examples for the use of Major and Minor Group field codes for this discipline. See CLG Sections 1.5 and 1.6 for complete rules and options governing the use of Major and Minor Group field codes.

### Architectural Discipline Designators

#### Designator	Description

A	Architectural

AD	Architectural Demolition

AE	Architectural Elements

AF	Architectural Finishes

AG	Architectural Graphics

AI	Architectural Interiors

AS	Architectural Site

AJ	User Defined

AK	User Defined

### Architectural Layer List

#### Layer Name	Description

A□-AREA	Area

A□-AREA-OCCP	Area: occupant or employee names

A□-BARR	Barrier

A□-BARR-AIR~	Barrier: air

A□-CLNG	Ceiling

A□-CLNG-ACCS	Ceiling: access

A□-CLNG-GRID	Ceiling: grid

A□-CLNG-OPNG	Ceiling: openings

A□-CLNG-SUSP	Ceiling: suspended elements

A□-CLNG-TEES	Ceiling: main tees

A□-COLS	Columns

A□-CONV	Conveying systems

A□-DOOR	Doors

A□-DOOR-FULL	Doors: full-height (swing and leaf)

A□-DOOR-PRHT	Doors: partial-height (swing and leaf)

A□-EQPM	Equipment

A□-EQPM-ACCS	Equipment: access

A□-EQPM-FIXD	Equipment: fixed

A□-EQPM-OVHD	Equipment: overhead

A□-FLOR	Floor

A□-FLOR-CSWK	Floor: casework

A□-FLOR-EVTR	Floor: elevator cars and equipment

A□-FLOR-FIXT	Floor: fixtures (plumbing)

A□-FLOR-HRAL	Floor: handrails/guard rails

A□-FLOR-LEVL	Floor: level changes (ramps, pits, depressions)

A□-FLOR-OTLN	Floor: outline

A□-FLOR-OVHD	Floor: overhead

A□-FLOR-RAIS	Floor: raised

A□-FLOR-RISR	Floor: risers

A□-FLOR-SIGN	Floor: signage

A□-FLOR-SPCL	Floor: specialties (toilet room accessories, display cases) A□-FLOR-STRS	 Floor: stair treads (escalators, ladders)

A□-FLOR-TPTN	Floor: toilet partitions

A□-FLOR-WDWK	Floor: architectural woodwork

A□-FURN	Furnishings

A□-FURN-FILE	Furnishings: file cabinets

A□-FURN-FIXD	Furnishings: fixed

A□-FURN-FREE	Furnishings: freestanding

A□-FURN-PLNT	Furnishings: plants

A□-FURN-PNLS	Furnishings: system panels

A□-FURN-SEAT	Furnishings: seating

A□-FURN-STOR	Furnishings: storage (component system)

A□-FURN-WKSF	Furnishings: work surface (component system) A□-GLAZ	Glazing

A□-GLAZ-FULL	Glazing: full-height

A□-GLAZ-PRHT	Glazing: partial-height

A□-GLAZ-SILL	Glazing: window sills

A□-HVAC	HVAC systems

A□-HVAC-RDFF	HVAC systems: return air diffusers

A□-HVAC-SDFF	HVAC systems: supply diffusers

A□-LITE	Lighting

A□-ROOF	Roof

A□-ROOF-HRAL	Roof: handrails/guard rails

A□-ROOF-LEVL	Roof: level changes

A□-ROOF-OTLN	Roof: outline

A□-ROOF-RISR	Roof: risers

A□-ROOF-STRS	Roof: stair treads (ladders)

A□-WALL	Walls

A□-WALL-CAVI	Walls: cavity

A□-WALL-CNTR	Walls: center

A□-WALL-CURT	Walls: curtain

A□-WALL-FIRE	Walls: fire protection

A□-WALL-FULL	Walls: full-height

A□-WALL-FULL-EXTR	 Walls: full-height: exterior A□-WALL-FULL-INTR	Walls: full-height: interior

A□-WALL-HEAD	Walls: door and window headers

A□-WALL-JAMB	Walls: door and window jambs

A□-WALL-MESH	Walls: mesh or wire

A□-WALL-MOVE	Walls: moveable

A□-WALL-PATT	Walls: texture and hatch patterns

A□-WALL-PRHT	Walls: partial-height

## CIVIL LAYER LIST

### Civil Field Codes

The Layer Names shown below provide examples for the use of Major and Minor Group field codes for this discipline. See CLG Sections 1.5 and 1.6 for complete rules and options governing the use of Major and Minor Group field codes.

The Civil Discipline is defined as a project or a portion of a project that is usually contained within a single property boundary.

### Civil Discipline Designators

#### Designator	Description

C	Civil

CD	Civil Demolition

CG	Civil Grading

CI	Civil Improvements

CN	Civil Nodes

CP	Civil Paving

CS	Civil Site

CT	Civil Transportation

CU	Civil Utilities

CJ	User Defined

CK	User Defined

### Civil Layer List

#### Layer Name	Description

C□-AFLD	Airfields

C□-AFLD-ASPH	Airfields: asphalt

C□-AFLD-CNTR	Airfields: center

C□-AFLD-CONC	Airfields: concrete

C□-AFLD-FLNE	Airfields: fire lane

C□-AFLD-FLNE-MRKG	 Airfields: fire lane: pavement markings C□-AFLD-FLNE-SIGN	Airfields: fire lane: signage

C□-AFLD-GRVL	Airfields: gravel

C□-AFLD-MRKG	Airfields: pavement markings

C□-AFLD-SIGN	Airfields: signage

C□-AFLD-STAN	Airfields: stationing

C□-AFLD-WHIT	Airfields: white paint

C□-AFLD-WHIT-TICK	Airfields: white paint: tick marks C□-AFLD-YELO	Airfields: yellow paint

C□-AFLD-YELO-TICK	Airfields: yellow paint: tick marks C□-BLDG	Buildings and primary structures

C□-BLDG-DECK	Buildings and primary structures: deck (attached, no roof

overhead)

C□-BLDG-OTLN	Buildings and primary structures: outline

C□-BLDG-OVHD	Buildings and primary structures: overhead

C□-BLDG-PRCH	Buildings and primary structures: porch (attached, roof

overhead)

C□-BLIN	Baseline

C□-BLIN-STAN	Baseline: stationing

C□-BORE	Borings

C□-BRDG	Bridge

C□-BRDG-CNTJ	Bridge: construction joint

C□-BRDG-CNTR	Bridge: center

C□-BRDG-DECK	Bridge: deck

C□-BRDG-EXPJ	Bridge: expansion joint

C□-BRDG-FALT	Bridge: fault/break line

C□-BRDG-HIDD	Bridge: objects or lines hidden from view

C□-BRDG-OBJT	Bridge: objects

C□-BRDG-OBJT-PRIM	Bridge: objects: primary C□-BRDG-OBJT-SECD	 Bridge: objects: secondary C□-BRDG-RBAR	Bridge: reinforcing bar

C□-CATV	Cable television system

C□-CATV-OVHD	Cable television system: overhead

C□-CATV-POLE	Cable television system: pole

C□-CATV-UGND	Cable television system: underground

C□-CEME	Cemetery

C□-CHAN	Navigable channels

C□-CHAN-BWTR	Navigable channels: breakwater

C□-CHAN-CNTR	Navigable channels: center

C□-CHAN-DACL	Navigable channels: de-authorized channel limits, anchorages,

etc.

C□-CHAN-DOCK	Navigable channels: decks, docks, floats, piers C□-CHAN-NAID	Navigable channels: navigation aids

C□-COMM	Communications

C□-COMM-OVHD	Communications: overhead

C□-COMM-POLE	Communications: pole

C□- COMM-UGND	Communications: underground

C□-CTRL	Control points

C□-CTR L-BMRK	Control points: benchmarks

C□-CTRL-FLYS	Control points: fly station

C□-CTRL-GRID	Control points: grid

C□-CTRL-HORZ	Control points: horizontal

C□-CTRL-HVPT	Control points: horizontal/vertical

C□-CTRL-PNPT	Control points: panel points

C□-CTRL-TRAV	Control points: transverse

C□-CTRL-VERT	Control points: vertical

C□-DFLD	Drain fields

C□-DFLD-OTLN	Drain fields: outline

C□-DFLD-PROF	Drain fields: profile

C□-DRIV	Driveways

C□-DRIV-ASPH	Driveways: asphalt

C□-DRIV-CNTR	Driveways: center

C□-DRIV-CONC	Driveways: concrete

C□-DRIV-CURB	Driveways: curb

C□-DRIV-CURB-BACK	Driveways: curb: back C□-DRIV-CURB-FACE	Driveways: curb: face C□-DRIV-FLNE	Driveways: fire lane

C□-DRIV-FLNE-MRKG	Driveways: fire lane: pavement markings

C□-DRIV-FLNE-SIGN	 Driveways: fire lane: signage C□-DRIV-GRVL	Driveways: gravel

C□-DRIV-MRKG	Driveways: pavement markings

C□-DRIV-SIGN	Driveways: signage

C□-DRI V-UPVD	Driveways: unpaved surface

C□-DRIV-WHIT	Driveways: white paint

C□-DRIV-WHIT-TICK	Driveways: white paint: tick marks C□-DRIV-YELO	 Driveways: yellow paint

C□-DRIV-YELO-TICK	Driveways: yellow paint: tick marks C□-DTCH	Ditches or washes

C□-DTCH-BOTM	Ditches or washes: bottom

C□-DTCH-CNTR	Ditches or washes: center

C□-DTCH-EWAT	Ditches or washes: edge of water

C□-DTCH-TOP~	Ditches or washes: top

C□-EROS	Erosion and sediment control

C□-EROS-CIPR	Erosion and sediment control: culvert inlet protection C□-EROS-CNTE	 Erosion and sediment control: construction entrance C□-EROS-DDIV	 Erosion and sediment control: drainage divides

C□-EROS-DVDK	 Erosion and sediment control: diversion dike C□-EROS-INPR	Erosion and sediment control: inlet protection

C□-EROS-SILT	Erosion and sediment control: silt fence

C□-EROS-SSLT	Erosion and sediment control: super silt fence

C□-ESMT	Easements

C□-ESMT-ACCS	Easements: access (pedestrian only; private access) C□-ESMT-CATV	Easements: utility - cable television system

C□-ESMT-CONS	Easements: conservation

C□-ESMT-CSTG	Easements: construction/grading

C□-ESMT-ELEC	Easements: electrical

C□-ESMT-FDPL	Easements: flood plain

C□-ESMT-INEG	Easements: ingress/egress (vehicles; private access) C□-ESMT-LSCP	Easements: landscape

C□-ESMT-NGAS	Easements: natural gas line

C□-ESMT-PHON	Easements: telephone line

C□-ESMT-ROAD	Easements: roadway

C□-ESMT-ROAD-PERM	Easements: roadway: permanent C□-ESMT-ROAD-TEMP	Easements: roadway: temporary

C□-ESMT-RWAY	Easements: right-of-way (public access)

C□-ESMT-SGHT	Easements: sight distance

C□-ESMT-SSWR	Easements: sanitary sewer

C□-ESMT-STRM	Easements: storm sewer

C□-ESMT-SWMT	Easements: storm water management

C□-ESMT-TRAL	Easements: trail or path (public access)

C□-ESMT-UTIL	Easements: utility lines

C□-ESMT-WATR	Easements: water supply

C□-FENC	Fences

C□-FENC-GRAL	Fences: guard rail

C□-FENC-POST	Fences: posts

C -FENC-STEL	Fences: steel (barbed wire and/or chain link)

C□-FENC-WOOD	Fences: wood

C□-FIRE	Fire protection

C□-FIRE-HYDT	Fire protection: hydrants and connections

C□-FIRE-PIPE	Fire protection: piping

C□-FIRE-UGND	Fire protection: underground

C□-FLHA	Flood hazard area

C□-FLHA-025Y	Flood hazard area: 25 year mark

C□-FLHA-050Y	Flood hazard area: 50 year mark

C□-FLHA-100Y	Flood hazard area: 100 year mark

C□-FLHA-200Y	Flood hazard area: 200 year mark

C□-FUEL	Fuel systems

C□-FUEL-EQPM	Fuel systems: equipment (pumps, motors)

C□-FUEL-INST	Fuel systems: instrumentation (meters, valves, etc.)

C□-FUEL-MHOL	Fuel systems: manhole

C□-FUEL-PIPE	Fuel systems: piping

C□-FUEL-TANK	Fuel systems: storage tanks

C□-FUEL-UGND	Fuel systems: underground

C□-HYDR	Hydraulic structure

C□-HYDR-BAFL	Hydraulic structure: baffle block and splash pad C□-HYDR-BASN	Hydraulic structure: stilling and settling basins

C□-HYDR-CNDT	Hydraulic structure: diversion/bypass conduits/culvers C□-HYDR-COFF	 Hydraulic structure: coffer dam

C□-HYDR-DAM~	Hydraulic structure: dam

C□-HYDR-FISH	Hydraulic structure: fish ladder/passage

C□-HYDR-FLUM	Hydraulic structure: flume

C□-HYDR-INTK	Hydraulic structure: intake

C□-HYDR-NOVR	Hydraulic structure: non-overflow structure C□-HYDR-PENS	Hydraulic structure: penstock

C□-LOCN	Limits of construction

C□-NGAS	Natural gas systems

C□-NGAS-EQPM	Natural gas systems: equipment (pumps, motors)

C□-NGAS-INST	Natural gas systems: instrumentation (meters, valves, etc.)

C□-NGAS-MHOL	Natural gas systems: manhole

C□-NGAS-PIPE	Natural gas systems: piping

C□-NGAS-TANK	Natural gas systems: storage tanks

C□-NGAS-UGND	Natural gas systems: underground

C□-PERC	Perc testing

C□-PERC-HOLE	Perc testing: holes

C□-POND	Ponds

C□-POND-EDGE	Ponds: edge

C□-POND-SWAY	Ponds: spillway

C□-POND-TOPB	Ponds: top of bank

C□-POWR	Power

C□-POWR-FENC	Power: fences

C□-POWR-INST	 Power: instrumentation (meters, transformers) C□-POWR-MHOL	Power: manhole

C□-POWR-OVHD	Power: overhead

C□-POWR-POLE	Power: pole

C□-POWR-STRC	Power: structures

C□-POWR-UGND	Power: underground

C□-PRKG	Parking lots

C□-PRKG-ASPH	Parking lots: asphalt

C□-PRKG-CARS	Parking lots: cars and other vehicles

C□-PRKG-CONC	Parking lots: concrete

C□-PRKG-CURB	Parking lots: curb

C□-PRKG-CURB-BACK	Parking lots: curb: back C□-PRKG-CURB-FACE	Parking lots: curb: face

C□-PRKG-DRAN	Parking lots: drainage slope indications

C□-PRKG-FIXT	 Parking lots: fixtures (wheel stops, parking meters, etc.) C□-PRKG-FLNE	Parking lots: fire lane

C□-PRKG-FLNE-MRKG	Parking lots: fire lane: pavement markings C□-PRKG-FLNE-SIGN	Parking lots: fire lane: signage

C□-PRKG-GRVL	Parking lots: gravel

C□-PRKG-MRKG	Parking lots: pavement markings

C□-PRKG-SIGN	Parking lots: signage

C□-PRKG-STRP	Parking lots: striping

C□-PRKG-UPVD	Parking lots: unpaved surface

C□-PRKG-WHIT	Parking lots: white paint

C□-PRKG-WHIT-TICK	Parking lots: white paint: tick marks C□-PRKG-YELO	Parking lots: yellow paint

C□-PRKG-YELO-TICK	Parking lots: yellow paint: tick marks C□-PROP	Property

C□-PROP-LINE	Property: lines

C□-PROP-SBCK	Property: setback lines

C□-PVMT	Pavement

C□-PVMT-ASPH	Pavement: asphalt

C□-PVMT-CONC	Pavement: concrete

C□-PVMT-GRVL	Pavement: gravel

C□-RAIL	Railroad

C□-RAIL-CNTR	Railroad: center

C□-RAIL-EQPM	Railroad: equipment (gates, signals, etc.)

C□-RAIL-TRAK	Railroad: track

C□-RIVR	River

C□-RIVR-BOTM	River: bottom

C□-RIVR-CNTR	River: center

C□-RIVR-EDGE	River: edge

C□-RIVR-TOPB	River: top of bank

C□-ROAD	Roadways

C□-ROAD-ASPH	Roadways: asphalt

C□-ROAD-CNTR	Roadways: center

C□-ROAD-CONC	Roadways: concrete

C□-ROAD-CURB	Roadways: curb

C□-ROAD-CURB-BACK	Roadways: curb: back C□-ROAD-CURB-FACE	Roadways: curb: face C□-ROAD-FLNE	Roadways: fire lane

C□-ROAD-FLNE-MRKG	Roadways: fire lane: pavement markings C□-ROAD-FLNE-SIGN	Roadways: fire lane: signage

C□-ROAD-GRVL	Roadways: gravel

C□-ROAD-MRKG	Roadways: pavement markings

C□-ROAD-PROF	Roadways: profile

C□-ROAD-SIGN	Roadways: signage

C□-ROAD-STAN	Roadways: stationing

C□-ROAD-UPVD	Roadways: unpaved surface

C□-ROAD-WHIT	Roadways: white paint

C□-ROAD-WHIT-TICK	 Roadways: white paint: tick marks C□-ROAD-YELO	Roadways: yellow paint

C□-ROAD-YELO-TICK	Roadways: yellow paint: tick marks C□-RRAP	Riprap

C□-SGHT	Sight distance

C□-SGHT-PROF	Sight distance: profile

C□-SOIL	Soils

C□-SSWR	Sanitary sewer

C□-SSWR-DIAG	Sanitary sewer: diagrams

C□-SSWR-FORC	Sanitary sewer: force main

C□-SSWR-LATL	Sanitary sewer: lateral line

C□-SSWR-MHOL	Sanitary sewer: manhole

C□-SSWR-PIPE	Sanitary sewer: piping

C□-SSWR-PIPE-RCON	 Sanitary sewer: piping: reinforced concrete C□-SSWR-PIPE-STEL	Sanitary sewer: piping: steel

C□-SSWR-PROF	Sanitary sewer: profile

C□-SSWR-STAN	Sanitary sewer: stationing

C□-SSWR-STRC	Sanitary sewer: structures

C□-SSWR-UGND	Sanitary sewer: underground

C□-STEM	Steam system

C□-STEM-INST	Steam system: instrumentation (meters, valves, etc.) C□-STEM-MHOL	 Steam system: manhole

C□-STEM-PIPE	Steam system: piping

C□-STEM-STRC	Steam system: structures

C□-STEM-UGND	Steam system: underground

C□-STRM	Storm sewer

C□-STRM-CNTR	Storm sewer: center

C□-STRM-DIAG	Storm sewer: diagrams

C□-STRM-HWAL	Storm sewer: headwall

C□-STRM-MHOL	Storm sewer: manhole

C□-STRM-PIPE	Storm sewer: piping

C□-STRM-PIPE-CMTL	 Storm sewer: piping: corrugated metal C□-STRM-PIPE-RCON	 Storm sewer: piping: reinforced concrete C□-STRM-PROF	Storm sewer: profile

C□-STRM-STAN	Storm sewer: stationing

C□-STRM-STRC	Storm sewer: structures

C□-STRM-UGND	Storm sewer: underground

C□-SWLK	Sidewalks

C□-SWLK-ASPH	Sidewalks: asphalt

C□-SWLK-CONC	Sidewalks: concrete

C□-TINN	Triangulated irregular network

C□-TINN-BNDY	Triangulated irregular network: boundary

C□-TINN-FALT	Triangulated irregular network: fault/break lines

C□-TINN-VIEW	Triangulated irregular network: triangulation view

C□-TINN-VOID	Triangulated irregular network: void regions

C□-TOPO	Topographic feature

C□-TOPO-DEPR	Topographic feature: depression

C□-TOPO-MAJR	Topographic feature: major (contours)

C□-TOPO-MINR	Topographic feature: minor (contours)

C□-TOPO-SPOT	Topographic feature: spot elevations

C□-TOPO-TPIT	Topographic feature: test pits

C□-TRAL	Trails or paths

C□-TRAL-ASPH	Trails or paths: asphalt

C□-TRAL-CONC	Trails or paths: concrete

C□-TRAL-GRVL	Trails or paths: gravel

C□-TRAL-MRKG	Trails or paths: pavement markings

C□-TRAL-SIGN	Trails or paths: signage

C□-TRAL-UPVD	Trails or paths: unpaved surface

C□-WALL	Walls

C□-WALL-CTLJ	Walls: control joint

C□-WALL-NSBR	Walls: noise barrier

C□-WALL-RTWL	Walls: retaining wall

C□-WALL-SHEA	Walls: structural bearing or shear walls

C□-WATR	Water supply

C□-WATR-DIAG	Water supply: diagrams

C□-WATR-INST	 Water supply: instrumentation (meters, valves, etc.) C□-WATR-PIPE	Water supply: piping

C□-WATR-PROF	Water supply: profile

C□-WATR-STAN	Water supply: stationing

C□-WATR-STRC	Water supply: structures

C□-WATR-UGND	Water supply: underground

C□-WATR-WELL	Water supply: well

C□-WETL	Wetlands

C□-WWAY	Waterway

C□-WWAY-DLPH	Waterway: dolphin

C□-WWAY-FEND	Waterway: fender

C□-WWAY-MOOR	Waterway: mooring

## CONTRACTOR/SHOP DRAWING LAYER LIST

### Contractor/Shop Drawing Field Codes

The Layer Names shown below provide examples for the use of Major and Minor Group field codes for this discipline. See CLG Sections 1.5 and 1.6 for complete rules and options governing the use of Major and Minor Group field codes.

### Contractor/Shop Drawing Discipline Designators

#### Designator	Description

Z	Contractor/Shop Drawings

ZJ	User Defined

ZK	User Defined

### Contractor/Shop Drawing Layer List

#### Layer Name	Description

No layer names have been prescribed for this discipline.

## ELECTRICAL LAYER LIST

### Electrical Field Codes

The Layer Names shown below provide examples for the use of Major and Minor Group field codes for this discipline. See CLG Sections 1.5 and 1.6 for complete rules and options governing the use of Major and Minor Group field codes.

#### Designator	Description

ED	Electrical Demolition

EI	Electrical Instrumentation

EL	Electrical Lighting

EP	Electrical Power

ES	Electrical Site

ET	Electrical Telecommunications

EY	Electrical Auxiliary Systems

EJ	User Defined

EK	User Defined

### Electrical Layer List

#### Layer Name	Description

E□-ALRM	Alarm system

E□-AREA	Area

E□-AREA-OFST	Area: offset zones

E□-AREA-SHAD	Area: shade zones

E□-AUXL	Auxiliary systems

E□-BELL	Bell system

E□-CABL	Cable systems

E□-CABL-COAX	Cable systems: coax cable

E□-CABL-FIBR	Cable systems: fiber optics cable

E□-CABL-MULT	Cable systems: multi-conductor cable

E□-CABL-TRAY	Cable systems: cabletray and wireways

E□-CATH	Cathodic protection system

E□-CATH-ANOD	Cathodic protection system: sacrificial anode E□-CATH-CURR	Cathodic protection system: impress current E□-CATH-TEST	 Cathodic protection system: test stations

E□-CCTV	Closed-circuit television system

E□-CLOK	Clock system

E□-CLOK-CIRC	Clock system: circuits

E□-CLOK-CLNG	Clock system: ceiling

E□-CLOK-CNMB	Clock system: circuit number

E□-CLOK-EQPM	Clock system: equipment

E□-CLOK-FLOR	Clock system: floor

E□-CLOK-WALL	Clock system: wall

E□-COMM	Communications

E□-COMM-CIRC	Communications: circuits

E□-COMM-CLNG	Communications: ceiling

E□-COMM-CNMB	Communications: circuit number

E□-COMM-EQPM	Communications: equipment

E□-COMM-WALL	Communications: wall

E□-CONT	Controls and instrumentation

E□-CONT DEVC	Controls and instrumentation: devices

E□-CONT-WIRE	Controls and instrumentation: wiring

E□-DATA	Data/LAN system

E□-DATA-CIRC	Data/LAN system: circuits

E□-DATA-CLNG	Data/LAN system: ceiling

E□-DATA-CNMB	Data/LAN system: circuit number

E□-DATA-EQPM	Data/LAN system: equipment

E□-DATA-FLOR	Data/LAN system: floor

E□-DATA-WALL	Data/LAN system: wall

E□-DIAG	Diagrams

E□-DIAG-BKRS	Diagrams: breakers

E□-DIAG-BUSS	Diagrams: bus duct

E□-DIAG-ENCL	Diagrams: equipment enclosures

E□-DIAG-EQPM	Diagrams: equipment

E□-DIAG-FEED	Diagrams: feeders

E□-DIAG-FLOR	Diagrams: floor

E□-DIAG-GRND	Diagrams: ground

E□-DIAG-SWCH	Diagrams: switches

E□-DIAG-XFMR	Diagrams: transformers

E□-DICT	Dictation system

E□-DICT-CIRC	Dictation system: circuits

E□-DICT-CLNG	Dictation system: ceiling

E□-DICT-CNMB	Dictation system: circuit number

E□-DICT-EQPM	Dictation system: equipment

E□-DICT-WALL	Dictation system: wall

E□-FIRE	Fire protection

E□-FIRE-BARR	Fire protection: barrier

E□-FIRE-CIRC	Fire protection: circuits

E□-FIRE-CLNG	Fire protection: ceiling

E□-FIRE-CNMB	Fire protection: circuit number

E□-FIRE-EQPM	Fire protection: equipment

E□-FIRE-WALL	Fire protection: wall

E□-GRND	Ground system

E□-GRND-CIRC	Ground system: circuits

E□-GRND-CLNG	Ground system: ceiling

E□-GRND-CNMB	Ground system: circuit number

E□-GRND-DIAG	Ground system: diagrams

E□-GRND-EQPM	Ground system: equipment

E□-GRND-EQUI	Ground system: equipotential

E□-GRND-WALL	Ground system: wall

E□-INST	Instrumentation system

E□-INST-CIRC	Instrumentation system: circuits

E□-INST-CLNG	Instrumentation system: ceiling

E□-INST-CNMB	Instrumentation system: circuit number

E□-INST-EQPM	Instrumentation system: equipment

E□-INST-WALL	Instrumentation system: wall

E□-INTC	Intercom/PA systems

E□-LITE	Lighting

E□-LITE-CIRC	Lighting: circuits

E□-LITE-CIRC-CRIT	Lighting: circuits: critical

E□-LITE-CIRC-EMER	Lighting: circuits: emergency E□-LITE-CLNG	Lighting: ceiling

E□-LITE-CLNG-CRIT	Lighting: ceiling: critical

E□-LITE-CLNG-EMER	Lighting: ceiling: emergency E□-LITE-CLNG-EXIT	Lighting: ceiling: exit

E□-LITE-CNMB	Lighting: circuit number

E□-LITE-CNMB-CRIT	Lighting: circuit number: critical

E□-LITE-CNMB-EMER	Lighting: circuit number: emergency E□-LITE-EMER	Lighting: emergency

E□-LITE-EQPM	Lighting: equipment

E□-LITE-EQPM-CRIT	Lighting: equipment: critical

E□-LITE-EQPM-EMER	Lighting: equipment: emergency E□-LITE-EXIT	Lighting: exit

E□-LITE-EXTR	Lighting: exterior

E□-LITE-FLOR	Lighting: floor

E□-LITE-JBOX	Lighting: junction box

E□-LITE-OTLN	Lighting: outline

E□-LITE-ROOF	Lighting: roof

E□-LITE-SPCL	Lighting: special

E□-LITE-SWCH	Lighting: switches

E□-LITE-SWCH-CRIT	Lighting: switches: critical

E□-LITE-SWCH-EMER	 Lighting: switches: emergency E□-LITE-WALL	Lighting: wall

E□-LITE-WALL-CRIT	Lighting: wall: critical

E□-LITE-WALL-EMER	Lighting: wall: emergency E□-LITE-WALL-EXIT	 Lighting: wall: exit

E□-LTNG	Lightning protection system

E□-LTNG-CIRC	Lightning protection system: circuits

E□-LTNG-CLNG	Lightning protection system: ceiling

E□-LTNG-CNMB	Lightning protection system: circuit number

E□-LTNG-EQPM	Lightning protection system: equipment

E□-LTNG-WALL	Lightning protection system: wall

E□-MNTG	Mounting system

E□-NURS	Nurse call system

E□-NURS-CIRC	Nurse call system: circuits

E□-NURS-CLNG	Nurse call system: ceiling

E□-NURS-CNMB	Nurse call system: circuit number

E□-NURS-EQPM	Nurse call system: equipment

E□-NURS-FLOR	Nurse call system: floor

E□-NURS-WALL	Nurse call system: wall

E□-OBST	Obstructions

E□-PGNG	Paging system

E□-POWR	Power

E□-POWR-BUSW	Power: busways

E□-POWR-CABL	Power: cable systems

E□-POWR-CBOX	Power: combiner box

E□-POWR-CBOX-FTPT	Power: combiner box: area footprints E□-POWR-CIRC	Power: circuits

E□-POWR-CIRC-CRIT	Power: circuits: critical E□-POWR-CLNG	 Power: ceiling

E□-POWR-CLNG-CRIT	Power: ceiling: critical E□-POWR-CNDT	Power: conduit

E□-POWR-CNMB	Power: circuit number

E□-POWR-CNMB-CRIT	 Power: circuit number: critical E□-POWR-DEVC	Power: devices

E□-POWR-DSCO	Power: disconnect switches

E□-POWR-DSCO-ACFU	 Power: disconnect switches: fused ac E□-POWR-DSCO-ACNF	Power: disconnect switches: unfused ac E□-POWR-DSCO-DCFU	 Power: disconnect switches: fused dc E□-POWR-DSCO-DCNF	 Power: disconnect switches: unfused dc E□-POWR-EQPM	Power: equipment

E□-POWR-EQPM-CRIT	Power: equipment: critical E□-POWR-EXTR	Power: exterior

E□-POWR-FEED	Power: feeders

E□-POWR-FLOR	Power: floor

E□-POWR-FLOR-CRIT	Power: floor: critical E□-POWR-JBOX	Power: junction box

E□-POWR-PANL	Power: panels

E□-POWR-POCC	Power: point of common coupling

E□-POWR-POI~	Power: point of interconnection

E□-POWR-ROOF	Power: roof

E□-POWR-SWBD	Power: switchboards

E□-POWR-UCPT	Power: under-carpet wiring

E□-POWR-URAC	Power: underfloor raceways

E□-POWR-WALL	Power: wall

E□-POWR-WALL-CRIT	Power: wall: critical

E□-POWR-XFMR-PADM	 Power: transformers: pad-mounted E□-POWR-XFMR-POLM	 Power: transformers: pole-mounted E□-PVMD	Photovoltaic modules

E□-SITE	Site features

E□-SITE-OVHD	Site features: overhead

E□-SITE-POLE	Site features: pole

E□-SITE-UGND	Site features: underground

E□-SOUN	Sound system

E□-UTIL	Utilities

  - DISTRIBUTED ENERGY LAYER LIST

### Distributed Energy Field Codes

The Layer Names shown below provide examples for the use of Major and Minor Group field codes for this discipline. See CLG Sections 1.5 and 1.6 for complete rules and options governing the use of Major and Minor Group field codes.

### Distributed Energy Discipline Designators

#### Designator	Description

W	Distributed Energy

WC	Distributed Energy Civil

WD	Distributed Energy Demolition

WI	Distributed Energy Interconnection

WP	Distributed Energy Power

WS	Distributed Energy Structural

WT	Distributed Energy Telecommunications

WY	Distributed Energy Auxiliary Systems

WJ	User Defined

WK	User Defined

### Distributed Energy Layer List

#### Layer Name	Description

No layer names have been prescribed for this discipline.

## EQUIPMENT LAYER LIST

### Equipment Field Codes

The Layer Names shown below provide examples for the use of Major and Minor Group field codes for this discipline. See CLG Sections 1.5 and 1.6 for complete rules and options governing the use of Major and Minor Group field codes.

### Equipment Discipline Designators

#### Designator	Description

Q	Equipment

QA	Equipment Athletic

QB	Equipment Bank

QC	Equipment Dry Cleaning

QD	Equipment Detention

QE	Equipment Educational

QF	Equipment Food Service

QH	Equipment Hospital

QL	Equipment Laboratory

QM	Equipment Maintenance

QP	Equipment Parking Lot

QR	Equipment Retail

QS	Equipment Site

QT	Equipment Theatrical

QV	Equipment Video/Photographic

QY	Equipment Security

QJ	User Defined

QK	User Defined

### Equipment Layer List

#### Layer Name	Description

Q□-CMPR	Computer

Q□-CSWK	Casework

Q□-CSWK-DVDR	Casework: thin dividers

Q□-CSWK-EDGE	Casework: edge

Q□-CSWK-ELEV	Casework: elevation

Q□-CSWK-FIXT	Casework: fixtures (plumbing/service)

Q□-CSWK-FRMG	Casework: structural framing

Q□-CSWK-FULL	Casework: full-height (cabinets/lockers)

Q□-CSWK-GLAZ	Casework: glazing

Q□-CSWK-GRND	Casework: ground

Q□-CSWK-HRDW	Casework: hardware

Q□-CSWK-LOWR	Casework: lower (cabinets)

Q□-CSWK-PATT	Casework: texture and hatch patterns

Q□-CSWK-SHLF	Casework: wall mounted shelving

Q□-CSWK-SUBA	Casework: cabinet sub-assemblies, drawer boxes Q□-CSWK-UCTR	 Casework: undercounter (cabinets-for layout)

Q□-CSWK-UPPR	Casework: upper (cabinets)

Q□-CSWK-WKSF	Casework: work surface

Q□-ELEV	Elevation

Q□-ELEV-EQPM	Elevation: equipment

Q□-ELEV-FIXT	Elevation: fixtures (plumbing/service)

Q□-ELEV-GLAZ	Elevation: glazing

Q□-ELEV-HRDW	Elevation: hardware

Q□-ELEV-OVHD	Elevation: overhead

Q□-ELEV-PATT	Elevation: texture and hatch patterns

Q□-ELEV-STRC	Elevation: structures (support components)

Q□-EXHS	Exhaust system

Q□-MAJQ	Major equipment

Q□-MAJQ-ACCS	Major equipment: access

Q□-MAJQ-ENGR	Major equipment: engineering information

Q□-MAJQ-FIXD	Major equipment: fixed

Q□-MAJQ-MOVE	Major equipment: movable

Q□-MAJQ-MVNG	Major equipment: moving or suspended

Q□-MAJQ-OVHD	Major equipment: overhead

Q□-MAJQ-PATT	Major equipment: texture and hatch patterns

Q□-MAJQ-UCTR	Major equipment: undercounter

Q□-MINQ	Minor equipment

Q□-POWR	Power

Q□-SPCL	Special

Q□-SPCL-ACCS	Special: access

Q□-SPCL-ENGR	Special: engineering information

Q□-SPCL-FIXD	Special: fixed

Q□-SPCL-MOVE	Special: movable

Q□-SPCL-MVNG	Special: moving or suspended

Q□-SPCL-OVHD	Special: overhead

Q□-SPCL-PATT	Special: texture and hatch patterns

Q□-SPCL-UCTR	Special: undercounter

## FIRE PROTECTION LAYER LIST

### Fire Protection Field Codes

The Layer Names shown below provide examples for the use of Major and Minor Group field codes for this discipline. See CLG Sections 1.5 and 1.6 for complete rules and options governing the use of Major and Minor Group field codes.

### Fire Protection Discipline Designators

#### Designator	Description

F	Fire Protection

FA	Fire Detection and Alarm

FX	Fire Suppression

FJ	User Defined

FK	User Defined

### Fire Protection Layer List

#### Layer Name	Description

F□-AFFF	Aqueous film-forming foam system

F□-AFFF-EQPM	 Aqueous film-forming foam system: equipment F□-AFFF-PIPE	Aqueous film-forming foam system: piping

F□-CO2S	CO2 system

F□-CO2S-EQPM	CO2 system: equipment

F□-CO2S-PIPE	CO2 system: piping

F□-HALN	Halon

F□-HALN-EQPM	Halon: equipment

F□-HALN-PIPE	Halon: piping

F□-IGAS	Inert gas

F□-IGAS-EQPM	Inert gas: equipment

F□-IGAS-PIPE	Inert gas: piping

F□-PROT	Fire protection system

F□-PROT-ALRM	Fire protection system: alarm

F□-PROT-EQPM	Fire protection system: equipment

F□-PROT-EXTI	Fire protection system: extinguishers

F□-PROT-HOSE	Fire protection system: hoses

F□-PROT-HYDT	Fire protection: hydrants and connections

F□-PROT-RATE	Fire protection system: ratings

F□-PROT-RATE-DOOR	Fire protection system: ratings: doors F□-PROT-RATE-WALL	 Fire protection system: ratings: wall

F□-PROT-SMOK	 Fire protection system: smoke detector/heat sensors F□-SPKL	Sprinkler

F□-SPKL-C LHD	Sprinkler: ceiling heads

F□-SPKL-E QPM	Sprinkler: equipment

F□-SPKL-O THD	Sprinkler: other heads

F□-SPKL-PI PE	Sprinkler: piping

## GENERAL LAYER LIST

### General Field Codes

The Layer Names shown below provide examples for the use of Major and Minor Group field codes for this discipline. See CLG Sections 1.5 and 1.6 for complete rules and options governing the use of Major and Minor Group field codes.

### General Discipline Designators

#### Designator	Description

G	General

GC	General Contractual

GI	General Informational

GR	General Resource

GJ	User Defined

GK	User Defined

### General Layer List

#### Layer Name	Description

G□-ACCS	Access

G□-CODE	Code compliance plan

G□-EVAC	Evacuation plan

G□-FIRE	Fire protection plan

G□-PLAN	Key plan (floor plan)

G□-SITE	Key plan (site features)

## GEOTECHNICAL LAYER LIST

### Geotechnical Field Codes

The Layer Names shown below provide examples for the use of Major and Minor Group field codes for this discipline. See CLG Sections 1.5 and 1.6 for complete rules and options governing the use of Major and Minor Group field codes.

### Geotechnical Discipline Designators

#### Designator	Description

B	Geotechnical

BJ	User Defined

BK	User Defined

### Geotechnical Layer List

#### Layer Name	Description

B□-BORE	Borings

B□-BORE-FDTA	Borings: field data

B□-BORE-HOLE	Borings: holes (perc)

B□-BORE-LDTA	Borings: laboratory data

B□-DETL-ANNN	Detail: optional number

(A = letter, NNN = number between 001 and 999)

B□-DETL-ANNN-CONC	 Detail: optional number: concrete B□-DETL-ANNN-ERTH	Detail: optional number: earth

B□-DETL-ANNN-FDTA	Detail: optional number: field data

B□-DETL-ANNN-FILL	Detail: optional number: fill and cover material B□-DETL-ANNN-GENF	 Detail: optional number: general features

B□-DETL-ANNN-GNDW	 Detail: optional number: ground water B□-DETL-ANNN-LDTA	 Detail: optional number: laboratory data B□-DETL-ANNN-PVMT	Detail: optional number: pavement

B□-DETL-ANNN-SPCL	Detail: optional number: special

B□-DETL-ANNN-STRM	Detail: optional number: storm sewer

B□-DETL-ANNN-SUBS	Detail: optional number: sub-surface areas B□-DETL-ANNN-SURF	Detail: optional number: surface areas

## HAZARDOUS MATERIALS LAYER LIST

### Hazardous Materials Field Codes

The Layer Names shown below provide examples for the use of Major and Minor Group field codes for this discipline. See CLG Sections 1.5 and 1.6 for complete rules and options governing the use of Major and Minor Group field codes.

### Hazardous Materials Discipline Designators

#### Designator	Description

H	Hazardous Materials

HA	Hazardous Materials Asbestos

HC	Hazardous Materials Chemicals

HL	Hazardous Materials Lead

HP	Hazardous Materials PCB

HR	Hazardous Materials Refrigerants

HJ	User Defined

HK	User Defined

### Hazardous Materials Layer List

#### Layer Name	Description

H□-PLAN	Key plan (floor plan)

H□-SITE	Key plan (site features)

## INTERIORS LAYER LIST

### Interiors Field Codes

The Layer Names shown below provide examples for the use of Major and Minor Group field codes for this discipline. See CLG Sections 1.5 and 1.6 for complete rules and options governing the use of Major and Minor Group field codes.

### Interiors Discipline Designators

#### Designator	Description

I	Interior

ID	Interior Demolition

IF	Interior Furnishings

IG	Interior Graphics

IN	Interior Design

IJ	User Defined

IK	User Defined

### Interiors Layer List

#### Layer Name	Description

I□-AREA	Area

I□-AREA-OCCP	Area: occupant or employee names

I□-CLNG	Ceiling

I□-CLNG-ACCS	Ceiling: access

I□-CLNG-OPNG	Ceiling: openings

I□-CLNG-SUSP	Ceiling: suspended elements

I□-CLNG-TEES	Ceiling: main tees

I□-COLS	Columns

I□-CRPT	Carpet/carpet tile

I□-CSWK	Casework

I□-DOOR	Doors

I□-DOOR-FULL	Doors: full-height

I□-DOOR-PRHT	Doors: partial-height

I□-EQPM	Equipment

I□-EQPM-ACCS	Equipment: access

I□-EQPM-FIXD	Equipment: fixed

I□-EQPM-OVHD	Equipment: overhead

I□-EQPM-STOR	Equipment: storage

I□-FLOR	Floor

I□-FLOR-EVTR	Floor: elevator cars and equipment

I□-FLOR-FIXT	Floor: fixtures (plumbing)

I□-FLOR-HRAL	Floor: handrails/guard rails

I□-FLOR-LEVL	Floor: level changes (ramps, pits, depressions)

I□-FLOR-OTLN	Floor: outline

I□-FLOR-OVHD	Floor: overhead

I□-FLOR-RAIS	Floor: raised

I□-FLOR-RISR	Floor: risers

I□-FLOR-SIGN	Floor: signage

I□-FLOR-SPCL	Floor: architectural specialties (toilet room accessories, display

cases)

I□-FLOR-STRS	Floor: stair treads (escalators, ladders)

I□-FLOR-TPTN	Floor: toilet partitions

I□-FLOR-WDWK	Floor: architectural woodwork

I□-FNSH	Finishes

I□-FURN	Furnishings

I□-FURN-FILE	Furnishings: file cabinets

I□-FURN-FREE	Furnishings: freestanding

I□-FURN-PLNT	Furnishings: plants

I□-FURN-PNLS	Furnishings: system panels

I□-FURN-SEAT	Furnishings: seating

I□-FURN-STOR	Furnishings: storage (component system)

I□-FURN-WKSF	Furnishings: work surface (component system)

I□-GLAZ	Glazing

I□-GLAZ-FULL	Glazing: full-height

I□-GLAZ-PRHT	Glazing: partial-height

I□-GLAZ-SILL	Glazing: window sills

I□-HVAC	HVAC systems

I□-HVAC-RDFF	HVAC systems: return air diffusers

I□-HVAC-SDFF	HVAC systems: supply diffusers

I□-MILL	Millwork

I□-PRTN	Partitions

I□-PRTN-FIRE	Partitions: fire protection

I□-PRTN-FULL	Partitions: full-height

I□-PRTN-HEAD	Partitions: door and window headers

I□-PRTN-JAMB	Partitions: door and window jambs

I□-PRTN-MOVE	Partitions: moveable

I□-PRTN-PRHT	Partitions: partial-height

I□-TILE	Tile

## LANDSCAPE LAYER LIST

### Landscape Field Codes

The Layer Names shown below provide examples for the use of Major and Minor Group field codes for this discipline. See CLG Sections 1.5 and 1.6 for complete rules and options governing the use of Major and Minor Group field codes.

### Landscape Discipline Designators

#### Designator	Description

L	Landscape

LD	Landscape Demolition

LG	Landscape Grading

LI	Landscape Irrigation

LL	Landscape Lighting

LP	Landscape Planting

LR	Landscape Relocation

LS	Landscape Site

LJ	User Defined

LK	User Defined

### Landscape Layer List

#### Layer Name	Description

L□-FENC	Fences

L□-FENC-LINK	Fences: chain link

L□-FENC-LINK-04FT	 Fences: chain link: four feet high L□-FENC-LINK-06FT	 Fences chain link: six feet high L□-FENC-PRVC	Fences: privacy

L□-FENC-WOOD	Fences: wood

L□-IRRG	Irrigation

L□-IRRG-COVR	Irrigation: coverage

L□-IRRG-DRIP	Irrigation: drip irrigation tubing

L□-IRRG-EQPM	Irrigation: equipment (pumps, valves, and controllers) L□-IRRG-LTRL	Irrigation: lateral pipe

L□-IRRG-MAIN	Irrigation: mainline

L□-IRRG-PIPE	Irrigation: piping

L□-IRRG-SLVE	Irrigation: pipe sleeve

L□-IRRG-SPKL	Irrigation: sprinklers (rotors, heads)

L□-IRRG-VALV	Irrigation: valves

L□-PLNT	Plant and landscape material

L□-PLNT-BEDS	Plant and landscape material: perennial and annual beds L□-PLNT-BUSH	Plant and landscape material: bushes and shrubs

L□-PLNT-CONI	Plant and landscape material: coniferous trees

L□-PLNT-CTNR	Plant and landscape material: container or planter L□-PLNT-EDGR	Plant and landscape material: planting bed edger

L□-PLNT-EVGR	Plant and landscape material: evergreen trees - broadleaf L□-PLNT-GCVR	Plant and landscape material: ground cover

L□-PLNT-MLCH	Plant and landscape material: mulches - organic and inorganic L□-PLNT-PALM	Plant and landscape material: palm trees

L□-PLNT-PLNT	Plant and landscape material: plants

L□-PLNT-SEED	Plant and landscape material: seeding areas

L□-PLNT-SHAD	Plant and landscape material: shadow area

L□-PLNT-TREE	Plant and landscape material: trees

L□-PLNT-TURF	Plant and landscape material: lawn areas

L□-PLNT-VINE	Plant and landscape material: vines

L□-PVMT	Pavement

L□-PVMT-ASPH	Pavement: asphalt

L□-PVMT-BRCK	Pavement: brick

L□-PVMT-CONC	Pavement: concrete

L□-PVMT-CONC-AGGR	Pavement: concrete: exposed aggregate L□-PVMT-GRVL	Pavement: gravel

L□-PVMT-JNTC	Pavement: control joint

L□-PVMT-JNTE	Pavement: expansion joint (for concrete only)

L□-PVMT-PAVR	Pavement: unit pavers

L□-PVMT-RAMP	Pavement: accessible ramp

L□-PVMT-STRS	Pavement: stair treads

L□-SITE	Site features

L□-SITE-BRDG	Site features: bridge (pedestrian)

L□-SITE-CURB	Site features: curb

L□-SITE-CURB-BACK	 Site features: curb: back L□-SITE-CURB-FACE	Site features: curb: face

L□-SITE-DECK	Site features: deck (wood, typ.)

L□-SITE-FURN	Site features: furnishings

L□-SITE-PLAY	Site features: play structures

L□-SITE-PLAY-EQPM	 Site features: play structures: equipment L□-SITE-PLAY-ZONE	Site features: play structures: zoning

L□-SITE-POOL	Site features: pools and spas

L□-SITE-POOL-BACK	Site features: pools and spas: back of pool wall L□-SITE-POOL-FACE	Site features: pools and spas: face of pool wall L□-SITE-PRKG	Site features: parking

L□-SITE-PRKG-STRP	Site features : parking: striping

L□-SITE-ROAD	Site features: edge of roadway line

L□-SITE-ROCK	Site features: large rocks and rock outcroppings

L□-SITE-RRAP	Site features: riprap

L□-SITE-RTWL	Site features: retaining wall

L□-SITE-SPRT	Site features: sports fields

L□-SITE-SPRT-EQPM	 Site features: sports fields: equipment L□-SITE-SPRT-PERI	 Site features: sports fields: perimeter L□-SITE-STEP	Site features: steps

L□-SITE-SWLK	Site features: sidewalks and steps

L□-SITE-TRAL	Site features: trail or path

L□-SITE-TRAL-ASPH	 Site features: trail or path: asphalt L□-SITE-TRAL-CONC	 Site features: trail or path: concrete L□-SITE-TRAL-GRVL	Site features: trail or path: gravel L□-SITE-WALL	 Site features: walls

L□-SITE-WEIR	Site features: pool weir

L□-TOPO	Topographic feature

L□-TOPO-LIMI	Topographic feature: limit of earthwork

L□-TOPO-SPOT	Topographic feature: spot elevations

## MECHANICAL LAYER LIST

### Mechanical Field Codes

The Layer Names shown below provide examples for the use of Major and Minor Group field codes for this discipline. See CLG Sections 1.5 and 1.6 for complete rules and options governing the use of Major and Minor Group field codes.

### Mechanical Discipline Designators

#### Designator	Description

M	Mechanical

MD	Mechanical Demolition

MH	Mechanical HVAC

MI	Mechanical Instrumentation

MP	Mechanical Piping

MS	Mechanical Site

MJ	User Defined

MK	User Defined

### Mechanical Layer List

#### Layer Name	Description

M□-BRIN	Brine systems

M□-BRIN-EQPM	Brine systems: equipment

M□-BRIN-PIPE	Brine systems: piping

M□-CHIM	Chimneys and stacks

M□-CMPA	Compressed/processed air systems

M□-CMPA-EQPM	Compressed/processed air systems: equipment

M□-CMPA-PEQP	Compressed/processed air systems: process equipment M□-CMPA-PIPE	Compressed/processed air systems: piping

M□-CMPA-PPIP	Compressed/processed air systems: process piping M□-CNDW	Condenser water systems

M□-CNDW-EQPM	Condenser water systems: equipment M□-CNDW-PIPE	Condenser water systems: piping

M□-CNDW-RETN	Condenser water systems: return

M□-CNDW-RETN-PIPE	Condenser water systems: return: piping M□-CNDW-RETN-SKCH	Condenser water systems: return: sketch M□-CNDW-SPLY	Condenser water systems: supply

M□-CNDW-SPLY-PIPE	 Condenser water systems: supply: piping M□-CNDW-SPLY-SKCH	 Condenser water systems: supply: sketch M□-CONT	Controls and instrumentation

M□-CONT-THER	Controls and instrumentation: thermostats

M□-CONT-WIRE	 Controls and instrumentation: wiring (low voltage) M□-CWTR	Chilled water systems

M□-CWTR-CNDS	Chilled water systems: condensate piping M□-CWTR-EQPM	 Chilled water systems: equipment

M□-CWTR-PIPE	Chilled water systems: piping

M□-CWTR-RETN	Chilled water systems: return

M□-CWTR-RETN-PIPE	 Chilled water systems: return: piping M□-CWTR-RETN-SKCH	Chilled water systems: return: sketch M□-CWTR-SPLY	 Chilled water systems: supply

M□-CWTR-SPLY-PIPE	 Chilled water systems: supply: piping M□-CWTR-SPLY-SKCH	 Chilled water systems: supply: sketch M□-DOMW	Domestic water systems

M□-DOMW-MKUP	 Domestic water systems: make-up water M□-DUAL	Dual temperature systems

M□-DUAL-RETN	Dual temperature systems: return

M□-DUAL-RETN-PIPE	 Dual temperature systems: return: piping M□-DUAL-RETN-SKCH	Dual temperature systems: return: sketch M□-DUAL-SPLY	 Dual temperature systems: supply

M□-DUAL-SPLY-PIPE	Dual temperature systems: supply: piping M□-DUAL-SPLY-SKCH	Dual temperature systems: supply: sketch

M□-DUST	Dust and fume collection systems

M□-DUST-DUCT	Dust and fume collection systems: ductwork

M□-DUST-DUCT-CNTR	 Dust and fume collection systems: ductwork: center M□-DUST-EQPM	Dust and fume collection systems: equipment

M□-ELHT	Electric heat

M□-ELHT-EQPM	Electric heat: equipment

M□-ENER	Energy management systems

M□-ENER-EQPM	 Energy management systems: equipment M□-ENER-WIRE	Energy management systems: wiring

M□-EXHS	Exhaust system

M□-EXHS-CDFF	Exhaust system: ceiling diffusers

M□-EXHS-DUCT	Exhaust system: ductwork

M□-EXHS-DUCT-CNTR	Exhaust system: ductwork: center M□-EXHS-EQPM	 Exhaust system: equipment

M□-EXHS-RFEQ	Exhaust system: rooftop equipment

M□-FLOR	Floor

M□-FLOR-PENE	Floor: penetrations

M□-FUEL	Fuel systems

M□-FUEL-EQPM	Fuel systems: equipment

M□-FUEL-GGEP	Fuel systems: gas general piping

M□-FUEL-GGEP-HPIP	Fuel systems: gas general piping: high pressure piping M□-FUEL-GGEP-LPIP	Fuel systems: gas general piping: low-pressure piping M□-FUEL-GGEP-LQPG	Fuel systems: gas general piping: liquid petroleum gas

M□-FUEL-GGEP-MPIP	 Fuel systems: gas general piping: medium-pressure piping M□-FUEL-GPRP	Fuel systems: gas process piping

M□-FUEL-OPRP	Fuel systems: oil process piping

M□-FUEL-OGEP	Fuel systems: oil general piping

M□-FUEL-OGEP-DISC	 Fuel systems: oil general piping: discharge M□-FUEL-OGEP-FLLW	Fuel systems: oil general piping: flow

M□-FUEL-OGEP-GAGE	 Fuel systems: oil general piping: gauge M□-FUEL-OGEP-RETN	 Fuel systems: oil general piping: return M□-FUEL-OGEP-SPLY	 Fuel systems: oil general piping: supply M□-FUEL-OGEP-VENT	 Fuel systems: oil general piping: vents M□-FUME	Fume hood

M□-FUME-DUCT	Fume hood: ductwork

M□-FUME-EQPM	Fume hood: equipment

M□-GLYC	Glycol systems

M□-GLYC-RETN	Glycol systems: return

M□-GLYC-RETN-PIPE	Glycol systems: return: piping M□-GLYC-RETN-SKCH	Glycol systems: return: sketch

M□-GLYC-SPLY	Glycol systems: supply

M□-GLYC-SPLY-PIPE	 Glycol systems: supply: piping M□-GLYC-SPLY-SKCH	 Glycol systems: supply: sketch M□-HVAC	HVAC systems

M□-HVAC-BOXD	HVAC systems: mixing box, dual duct

M□-HVAC-BOXS	HVAC systems: mixing box, single duct

M□-HVAC-CDFF	HVAC systems: ceiling diffusers

M□-HVAC-CLDA	HVAC systems: cold air

M□-HVAC-CLDA-DUCT	HVAC systems: cold air: ductwork M□-HVAC-CLDA-EQPM	HVAC systems: cold air: equipment

M□-HVAC-CLDA-RSCH	HVAC systems: cold air: sketch line round or oval duct M□-HVAC-CLDA-SECT	HVAC systems: cold air: section

M□-HVAC-CLDA-SIZE	HVAC systems: cold air: ductwork size

M□-HVAC-CLDA-SSCH	HVAC systems: cold air: sketch line rectangular duct M□-HVAC-DMPR	HVAC systems: fire, smoke, volume damper

M□-HVAC-DOOR	HVAC systems: equipment doors

M□-HVAC-EFAN	HVAC systems: equipment with electric fans

M□-HVAC-EPDU	 HVAC systems: equipment with piping, ductwork and electricity M□-HVAC-EPIP	HVAC systems: equipment with piping and electricity

M□-HVAC-EQPM	HVAC systems: equipment

M□-HVAC-EXHS	HVAC systems: exhaust air

M□-HVAC-EXHS-DUCT	 HVAC systems: exhaust air: ductwork M□-HVAC-EXHS-EQPM	HVAC systems: exhaust air: equipment M□-HVAC-EXHS-GRIL	 HVAC systems: exhaust air: grilles

M□-HVAC-EXHS-RSCH	HVAC systems: exhaust air: sketch line round or oval duct M□-HVAC-EXHS-SECT	 HVAC systems: exhaust air: section

M□-HVAC-EXHS-SIZE	HVAC systems: exhaust air: ductwork size

M□-HVAC-EXHS-SSCH	HVAC systems: exhaust air: sketch line rectangular duct M□-HVAC-HOTA	 HVAC systems: hot air

M□-HVAC-HOTA-DUCT	 HVAC systems: hot air: ductwork M□-HVAC-HOTA-EQPM	HVAC systems: hot air: equipment

M□-HVAC-HOTA-RSCH	HVAC systems: hot air: sketch line round or oval duct M□-HVAC-HOTA-SECT	 HVAC systems: hot air: section

M□-HVAC-HOTA-SIZE	HVAC systems: hot air: ductwork size

M□-HVAC-HOTA-SSCH	HVAC systems: hot air: sketch line rectangular duct M□-HVAC-ODFF	 HVAC systems: other diffusers

M□-HVAC-PIPE	HVAC systems: piping

M□-HVAC-RDFF	HVAC systems: return air diffusers

M□-HVAC-RETN	HVAC systems: return

M□-HVAC-RETN-CNTR	HVAC systems: return: center

M□-HVAC-RETN-EQPM	HVAC systems: return: equipment

M□-HVAC-RETN-RSCH	HVAC systems: return: sketch line round or oval duct M□-HVAC-RETN-SECT	 HVAC systems: return: section

M□-HVAC-RETN-SIZE	HVAC systems: return: ductwork size

M□-HVAC-RETN-SSCH	HVAC systems: return: sketch line rectangular duct M□-HVAC-SDFF	 HVAC systems: supply diffusers

M□-HVAC-SPLY	HVAC systems: supply

M□-HVAC-SPLY-CNTR	HVAC systems: supply: center

M□-HVAC-SPLY-EQPM	HVAC systems: supply: equipment

M□-HVAC-SPLY-RSCH	HVAC systems: supply: sketch line round or oval duct M□-HVAC-SPLY-SECT	 HVAC systems: supply: section

M□-HVAC-SPLY-SIZE	HVAC systems: supply: ductwork size

M□-HVAC-SPLY-SSCH	 HVAC systems: supply: sketch line rectangular duct M□-HWTR	Hot water heating system

M□-HWTR-EQPM	Hot water heating system: equipment

M□-HWTR-PIPE	Hot water heating system: piping

M□-HWTR-RETN	Hot water heating system: return

M□-HWTR-RETN-PIPE	Hot water heating system: return: piping M□-HWTR-RETN: SKCH	Hot water heating system: return: sketch M□-HWTR-SPLY	Hot water heating system: supply

M□-HWTR-SPLY-PIPE	 Hot water heating system: supply: piping M□-HWTR-SPLY-SKCH	Hot water heating system: supply: sketch M□-LGAS	 Laboratory gas systems

M□-LGAS-EQPM	Laboratory gas systems: equipment

M□-LGAS-PIPE	Laboratory gas systems: piping

M□-MACH	Machine shop

M□-MDGS	Medical gas systems

M□-MDGS-CAIR	Medical gas systems: compressed air

M□-MDGS-EQPM	Medical gas systems: equipment

M□-MDGS-NITG	Medical gas systems: nitrogen

M□-MDGS-NOXG	Medical gas systems: nitrous oxide

M□-MDGS-OXYG	Medical gas systems: pure O2

M□-MDGS-PIPE	Medical gas systems: piping

M□-MDGS-SAIR	Medical gas systems: scavenge air

M□-MDGS-VACU	Medical gas systems: vacuum

M□-MKUP	Make-up air systems

M□-MKUP-CDFF	Make-up air systems: ceiling diffusers

M□-MKUP-DUCT	Make-up air systems: ductwork

M□-MKUP-EQPM	Make-up air systems: equipment

M□-MPIP	Miscellaneous piping systems

M□-MPIP-PIPE	Miscellaneous piping systems: piping

M□-NGAS	Natural gas systems

M□-NGAS-EQPM	Natural gas systems: equipment

M□-NGAS-PIPE	Natural gas systems: piping

M□-PROC	Process systems

M□-PROC-EQPM	Process systems: equipment

M□-PROC-PIPE	Process systems: piping

M□-RAIR	Relief air systems

M□-RCOV	Energy recovery systems

M□-RCOV-EQPM	Energy recovery systems: equipment

M□-RCOV-PIPE	Energy recovery systems: piping

M□-REFG	Refrigeration systems

M□-REFG-DISC	Refrigeration systems: discharge

M□-REFG-EQPM	Refrigeration systems: equipment

M□-REFG-PIPE	Refrigeration systems: piping

M□-REFG-RETN	Refrigeration systems: return

M□-REFG-SPLY	Refrigeration systems: supply

M□-ROOF	Roof

M□-ROOF-PENE	Roof: penetrations

M□-SMOK	Smoke extraction systems

M□-SMOK-CDFF	 Smoke extraction systems: ceiling diffusers M□-SMOK-DUCT	Smoke extraction systems: ductwork

M□-SMOK-EQPM	Smoke extraction systems: equipment

M□-SPCL	Special systems

M□-SPCL-EQPM	Special systems: equipment

M□-SPCL-PIPE	Special systems: piping

M□-STEM	Steam system

M□-STEM-BLBD	Steam system: boiler blow down piping

M□-STEM-BLBD-PIPE	 Steam system: boiler blow down piping: piping M□-STEM-CNDS	Steam system: condensate piping

M□-STEM-CNDS-SKCH	Steam system: condensate piping: sketch M□-STEM-EQPM	 Steam system: equipment

M□-STEM-HPIP	Steam system: high-pressure piping

M□-STEM-HPIP-SKCH	Steam system: high-pressure piping: sketch M□-STEM-LPIP	Steam system: low-pressure piping

M□-STEM-LPIP-SKCH	Steam system: low-pressure piping: sketch M□-STEM-MPIP	Steam system: medium-pressure piping

M□-STEM-MPIP-SKCH	Steam system: medium-pressure piping: sketch M□-TEST	Test equipment

M□-WALL	Wall

M□-WALL-PENE	Wall: penetrations

## OPERATIONS LAYER LIST

### Operations Field Codes

The Layer Names shown below provide examples for the use of Major and Minor Group field codes for this discipline. See CLG Sections 1.5 and 1.6 for complete rules and options governing the use of Major and Minor Group field codes.

### Operations Discipline Designators

#### Designator	Description

O	Operations

OJ	User Defined

OK	User Defined

### Operations Layer List

#### Layer Name	Description

No layer names have been prescribed for this discipline.

## PLUMBING LAYER LIST

### Plumbing Field Codes

The Layer Names shown below provide examples for the use of Major and Minor Group field codes for this discipline. See CLG Sections 1.5 and 1.6 for complete rules and options governing the use of Major and Minor Group field codes.

### Plumbing Discipline Designators

#### Designator	Description

P	Plumbing

PD	Plumbing Demolition

PL	Plumbing

PP	Plumbing Piping

PQ	Plumbing Equipment

PS	Plumbing Site

PJ	User Defined

PK	User Defined

### Plumbing Layer List

#### Layer Name	Description

P□-ACID	Acid waste systems

P□-ACID-EQPM	Acid waste systems: equipment

P□-ACID-PIPE	Acid waste systems: piping

P□-ACID-VENT	Acid waste systems: vents

P□-DOMW	Domestic water systems

P□-DOMW-CPIP	Domestic water systems: cold water piping

P□-DOMW-EQPM	Domestic water systems: equipment

P□-DOMW-HPIP	Domestic water systems: hot water piping

P□-DOMW-RISR	Domestic water systems: risers

P□-DOMW-RPIP	 Domestic water systems: recirculation piping P□-FLOR	Floor

P□-FLOR-PENE	Floor: penetrations

P□-MDGS	Medical gas systems

P□-MDGS-CAIR	Medical gas systems: compressed air

P□-MDGS-EQPM	Medical gas systems: equipment

P□-MDGS-NITG	Medical gas systems: nitrogen

P□-MDGS-NOXG	Medical gas systems: nitrous oxide

P□-MDGS-OXYG	Medical gas systems: pure O2

P□-MDGS-PIPE	Medical gas systems: piping

P□-MDGS-SAIR	Medical gas systems: scavenge air

P□-MDGS-VACU	Medical gas systems: vacuum

P□-ROOF	Roof

P□-ROOF-PENE	Roof: penetrations

P□-SSWR	Sanitary sewer

P□-SSWR-EQPM	Sanitary sewer: equipment

P□-SSWR-FIXT	Sanitary sewer: fixtures

P□-SSWR-FLDR	Sanitary sewer: floor drains

P□-SSWR-PIPE	Sanitary sewer: piping

P□-SSWR-RISR	Sanitary sewer: risers

P□-SSWR-VENT	Sanitary sewer: vents

P□-STRM	Storm sewer

P□-STRM-PIPE	Storm sewer: piping

P□-STRM-RFDR	Storm sewer: roof drains

P□-STRM-RISR	Storm sewer: risers

P□-WALL	Wall

P□-WALL-PENE	Wall: penetrations

## PROCESS LAYER LIST

### Process Field Codes

The Layer Names shown below provide examples for the use of Major and Minor Group field codes for this discipline. See CLG Sections 1.5 and 1.6 for complete rules and options governing the use of Major and Minor Group field codes.

### Process Discipline Designators

#### Designator	Description

D	Process

DA	Process Airs

DC	Process Chemicals

DD	Process Demolition

DE	Process Electrical

DG	Process Gases

DI	Process Instrumentation

DL	Process Liquids

DM	Process HPM Gases

DO	Process Oils

DP	Process Piping

DQ	Process Equipment

DR	Process Drains and Reclaims

DS	Process Site

DV	Process Vacuum

DW	Process Waters

DX	Process Exhaust

DY	Process Slurry

DJ	User Defined

DK	User Defined

### Process Layer List

#### Layer Name	Description

D□-AIR~-AA~~	Air: agitation air - system

D□-AIR~-BA~~	Air: breathable air - system

D□-AIR~-CA~~	Air: compressed air - system

D□-AIR~-CDA~	Air: clean dry air - system

D□-AIR~-HCDA	Air: high pressure clean dry air - system

D□-AIR~-IA~~	Air: instrument air - system

D□-AIR~-OA~~	Air: outside air - system

D□-AIR~-OFA~	Air: oil free air - system

D□-AIR~-PA~~	Air: plant air - system

D□-AIR~-V~~~	Air: vent - system

D□-CHEM-ARC~	Chemical: regenerative caustic - system

D□-CHEM-C~~~	Chemical: caustic - system

D□-CHEM-DEV~	Chemical: developer - system

D□-CHEM-EG~~	Chemical: ethylene glycol - system

D□-CHEM-H2O2	Chemical: hydrogen peroxide - system

D□-CHEM-HCL~	Chemical: hydrochloric acid - system

D□-CHEM-HF~~	Chemical: hydrofluoric acid - system

D□-CHEM-IPA~	Chemical: isopropyl alcohol - system

D□-CHEM-PHOS	Chemical: phosphoric acid - system

D□-CHEM-RER~	Chemical: solvent - system

D□-CHEM-SULF	Chemical: sulfuric acid - system

D□-CHEM-TMAH	Chemical: tmah - system

D□-DETL-BOLD	Detail: bold lines

D□-DETL-FINE	Detail: fine lines

D□-DETL-MEDM	Detail: medium lines

D□-DRAN-AMW~	Drains: ammonia waste - system

D□-DRAN-CD~~	Drains: condensate drain - system

D□-DRAN-CLW~	Drains: concentrated lead waste - system D□-DRAN-CMW~	Drains: concentrated metals waste - system D□-DRAN-CUPW	Drains: copper plating waste - system

D□-DRAN-CURW	Drains: copper rinse waste - system

D□-DRAN-CUSW	Drains: copper slurry waste - system

D□-DRAN-DIRC	Drains: DI reclaim - system

D□-DRAN-DLW~	Drains: dilute waste - system

D□-DRAN-EGW~	Drains: ethylene glycol waste - system

D□-DRAN-HFW~	Drains: hydrofluoric waste - system

D□-DRAN-IW~~	Drains: industrial waste - system

D□-DRAN-MW~~	Drains: metals waste - system

D□-DRAN-NPWR	Drains: non-potable water reuse - system D□-DRAN-OIW~	Drains: organic industrial waste - system

D□-DRAN-OLW~	Drains: organic liquid waste - system

D□-DRAN-OSW~	Drains: organic solvent waste - system

D□-DRAN-PHRC	Drains: phosphoric acid reclaim - system

D□-DRAN-PSW~	Drains: photo solvent waste - system

D□-DRAN-SDD~	Drains: scrubber duct drains - system

D□-DRAN-SLW~	Drains: slurry waste - system

D□-DRAN-SULF	Drains: sulfuric acid - system

D□-DRAN-SULR	Drains: sulfuric acid reclaim - system

D□-DRAN-SW~~	Drains: solvent waste - system

D□-DRAN-SWF~	Drains: solvent waste flammable - system

D□-DRAN-SWNF	Drains: solvent waste non-flammable - system D□-EXHS-AMEX	 Exhaust: ammonia exhaust - system

D□-EXHS-AREX	Exhaust: arsenic exhaust - system

D□-EXHS-HTEX	Exhaust: heat exhaust - system

D□-EXHS-SCEX	Exhaust: scrubber exhaust - system

D□-EXHS-SVEX	Exhaust: solvent exhaust - system

D□-GAS~-AR~~	Gas: argon - system

D□-GAS~-ARB~	Gas: argon bulk - system

D□-GAS~-BUT~	Gas: butane - system

D□-GAS~-CLG~	Gas: chlorine gas - system

D□-GAS~-H2~~	Gas: hydrogen - system

D□-GAS~-HE~~	Gas: helium - system

D□-GAS~-HPN2	Gas: high purity nitrogen - system

D□-GAS~-HPO2	Gas: high purity oxygen - system

D□-GAS~-LCHE	Gas: leak check helium - system

D□-GAS~-N2~~	Gas: nitrogen - system

D□-GAS~-N2O~	Gas: nitrous oxide - system

D□-GAS~-NG~~	Gas: natural gas - system

D□-GAS~-O2~~	Gas: oxygen - system

D□-GAS~-PRO~	Gas: propane - system

D□-GAS~-SG~~	Gas: specialty gas - system

D□-GAS~-UN2~	Gas: utility nitrogen - system

D□-GAS~-VN2~	Gas: venturi nitrogen - system

D□-GAS~-WAR~	Gas: weld argon - system

D□-LIQD-LPG~	Liquid: liquid petroleum gas - system

D□-OIL~-LO~~	Oil: lube oil - system

D□-PIPE	Piping

D□-PIPE-CNTR	Piping: center

D□-PIPE-EQPM	Piping: equipment

D□-PIPE-HDLN	Piping: hidden line

D□-PIPE-MISC	Piping: miscellaneous

D□-PIPE-PATT	Piping: texture and hatch patterns

D□-PIPE-UGND	Piping: underground

D□-SLUR-SLR~	Slurry: slurry return - system

D□-SLUR-SLS~	Slurry: slurry supply - system

D□-VACU-CLV~	Vacuum: chlorine vacuum - system

D□-VACU-CV~~	Vacuum: chemical vacuum - system

D□-VACU-EV~~	Vacuum: equipment vacuum - system

D□-VACU-HV~~	Vacuum: house vacuum - system

D□-VACU-HVA~	Vacuum: arsenic house vacuum - system

D□-VACU-PV~~	Vacuum: vacuum - system

D□-WATR-BFW~	Water: boiler feed water - system

D□-WATR-DIR~	Water: deionized water return - system

D□-WATR-DIS~	Water: deionized water supply - system

D□-WATR-DIWP	Water: DI polishing loop - system

D□-WATR-FW~~	Water: fire water - system

D□-WATR-HDIR	Water: hot DI return - system

D□-WATR-HDIS	Water: hot DI supply - system

D□-WATR-HDRC	Water: hot DI reclaim - system

D□-WATR-HPDR	Water: high pH DI return - system

D□-WATR-HPDS	Water: high pH DI supply - system

D□-WATR-ICW~	Water: industrial city water - system

D□-WATR-NPW~	Water: non-potable water - system

D□-WATR-PCWR	Water: cooling water return - system

D□-WATR-PCWS	Water: cooling water supply - system

D□-WATR-PW~~	Water: potable water - system

D□-WATR-RO~~	Water: reverse osmosis water - system

D□-WATR-ROR~	 Water: reverse osmosis reject water - system D□-WATR-TDIR	Water: tempered DI return - system

D□-WATR-TDIS	Water: tempered DI supply - system

D□-WATR-TW~~	Water: tempered water - system

D□-WATR-UPRW	 Water: ultra pure recycle water - system D□-WATR-UPW~	Water: ultra pure water - system

## RESOURCE LAYER LIST

### Resource Field Codes

The Layer Names shown below provide examples for the use of Major and Minor Group field codes for this discipline. See CLG Sections 1.5 and 1.6 for complete rules and options governing the use of Major and Minor Group field codes.

### Resource Discipline Designators

#### Designator	Description

R	Resource

RA	Resource Architectural

RC	Resource Civil

RE	Resource Electrical

RM	Resource Mechanical

RR	Resource Real Esate

RS	Resource Structural

RJ	User Defined

RK	User Defined

### Resource Layer List

#### ayer Name	Description

R□-INGR	Ingrants

R□-INGR-ESMT	Ingrants: easement

R□-INGR-LEAS	Ingrants: lease

R□-INGR- LICN	Ingrants: license

R□-INGR-PMIT	Ingrants: permit

R□-INGR-RSRV	Ingrants: reservation

R□-LAND	Land

R□-LAND-ALOC	Land: allocation

R□-LAND-CLAS	Land: classification

R□-OTGR	Outgrants

R□-OTGR-LEAS	Outgrants: lease

R□-OTGR-LICN	Outgrants: license

R□-OTGR-PMIT	Outgrants: permit

R□-OTGR-RSRV	Outgrants: reservation

R□-PROP	Property

R□-PROP-PRCL	Property: parcels

R□-PROP-TAKE	Property: taking lines

R□-PROP-TAKE-ELEV	 Property: taking lines: elevations R□-PROP-TRAC	Property: tract lines

R□-PROP-TRAC-DFEE	Property: tract lines: disposed fee R□-PROP-TRAC-FEE~	Property: tract lines: fee

R□-PROP-TRAC-LFEE	Property: tract lines: disposed less than fee R□-PROP-TRAC-NFEE	Property: tract lines: non-fee

## STRUCTURAL LAYER LIST

### Structural Field Codes

The Layer Names shown below provide examples for the use of Major and Minor Group field codes for this discipline. See CLG Sections 1.5 and 1.6 for complete rules and options governing the use of Major and Minor Group field codes.

### Structural Discipline Designators

#### Designator	Description

S	Structural

SB	Structural Substructure

SD	Structural Demolition

SF	Structural Framing

SS	Structural Site

SJ	User Defined

SK	User Defined

### Structural Layer List

#### Layer Name	Description

S□-ALGN	Alignment

S□-BEAM	Beams

S□-BEAM-ALUM	Beams: aluminum

S□-BEAM-CONC	Beams: concrete

S□-BEAM-STEL	Beams: steel

S□-BEAM-WOOD	Beams: wood

S□-BRCG	Bracing

S□-BRCG-ALUM	Bracing: aluminum

S□-BRCG-ALUM-HORZ	 Bracing: aluminum: horizontal S□-BRCG-ALUM-VERT	 Bracing: aluminum: vertical S□-BRCG-METL	Bracing: metal

S□-BRCG-STEL	Bracing: steel

S□-BRCG-STEL-HORZ	Bracing: steel: horizontal S□-BRCG-STEL-VERT	Bracing: steel: vertical S□-BRCG-WOOD	Bracing: wood

S□-BRCG-WOOD-HORZ	 Bracing: wood: horizontal S□-BRCG-WOOD-VERT	 Bracing: wood: vertical S□-COLS	Columns

S□-COLS-ABLT	Columns: anchor bolts

S□-COLS-ALUM	Columns: aluminum

S□-COLS-CONC	Columns: concrete

S□-COLS-STEL	Columns: steel

S□-COLS-WOOD	Columns: wood

S□-DECK	Deck

S□-DECK-FLOR	Deck: floor

S□-DECK-FLOR-OPNG	 Deck: floor: openings S□-DECK-ROOF	Deck: roof

S□-DECK-ROOF-OPNG	 Deck: roof: openings S□-DETL	Detail

S□-DETL-HSSS	Detail: hollow structural steel

S□-DETL-PLYW	Detail: plywood

S□-DETL-W2XS	Detail: dimension lumber

S□-FNDN	Foundation

S□-FNDN-FTNG	Foundation: footings

S□-FNDN-GRBM	Foundation: grade beams

S□-FNDN-PCAP	Foundation: pile caps

S□-FNDN-PIER	Foundation: drilled piers

S□-FNDN-PILE	Foundation: piles

S□-FNDN-RBAR	Foundation: reinforcing bar

S□-FNDN-RBAR-BOT1	Foundation: reinforcing bar: bottom group 1 S□-FNDN-RBAR-BOT2	Foundation: reinforcing bar: bottom group 2 S□-FNDN-RBAR-TOP1	 Foundation: reinforcing bar: top group 1

S□-FNDN-RBAR-TOP2	Foundation: reinforcing bar: top group 2 S□-FRAM	Braced frame or moment frame

S□-FSTN	Fasteners and connections

S□-GATE	Gate

S□-GRID	Grids

S□-GRID-EXTR	Grids: exterior

S□-GRID-INTR	Grids: interior

S□-GRLN	Grade line

S□-GRLN-SURF	Grade line: surface areas

S□-GRTG	Grating

S□-GRTG-OVHD	Grating: overhead

S□-HYDR	Hydraulic structure

S□-JNTS	Joints

S□-JNTS-CNTJ	Joints: construction joint

S□-JNTS-CTLJ	Joints: control joint

S□-JNTS-EXPJ	Joints: expansion joint

S□-JOIS	Joists

S□-JOIS-BRGX	Joists: bridging

S□-LNTL	Lintels

S□-PADS	Pads

S□-PADS-EQPM	Pads: equipment

S□-PLAT	Platform

S□-PLAT-FRMG	Platform: framing

S□-PLAT-GRTG	Platform: grating

S□-SIGN	Sign

S□-SIGN-BOUY	Sign: bouy

S□-SIGN-FRMG	Sign: framing

S□-SIGN-GAGE	Sign: gauge (staff)

S□-SIGN-TEXT	Sign: signage text

S□-SIGN-XTRU	Sign: extrusion

S□-SLAB	Slab

S□-SLAB-CONC	Slab: concrete

S□-SLAB-EDGE	Slab: edge

S□-SLAB-OPNG	Slab: openings (and depressions)

S□-SLAB-OPNX	Slab: opening indication ("x")

S□-SLAB-STEL	Slab: steel

S□-SLAB-WOOD	Slab: wood

S□-STIF	Stiffener

S□-STIF-LONG	Stiffener: longitudinal

S□-STIF-TRAV	Stiffener: transverse

S□-STRS	Stairs

S□-STRS-LADD	Stairs: ladders & ladder assemblies

S□-TRUS	Trusses

S□-WALL	Walls

S□-WALL-ABOV	Walls: above

S□-WALL-CMUW	Walls: concrete masonry unit

S□-WALL-CONC	Walls: concrete

S□-WALL-MSNW	Walls: masonry

S□-WALL-PCST	Walls: pre-cast concrete

S□-WALL-SHEA	Walls: structural bearing or shear walls

S□-WALL-STEL	Walls: steel stud

S□-WALL-VENR	Walls: veneer

S□-WALL-WOOD	Walls: wood

## SURVEY/MAPPING LAYER LIST

### Survey/Mapping Field Codes

The Layer Names shown below provide examples for the use of Major and Minor Group field codes for this discipline. See CLG Sections 1.5 and 1.6 for complete rules and options governing the use of Major and Minor Group field codes.

### Survey/Mapping Discipline Designators

#### Designator	Description

V	Survey/Mapping

VA	Survey/Mapping Aerial

VC	Survey/Mapping Computated Points

VF	Survey/Mapping Field

VI	Survey/Mapping Digital

VN	Survey/Mapping Node Points

VS	Survey/Mapping Staked Points

VU	Survey/Mapping Combined Utilities

VJ	User Defined

VK	User Defined

### Survey/Mapping Layer List

#### Layer Name	Description

V□-BLDG	Buildings and primary structures

V□-BLDG-DECK	Buildings and primary structures: deck (attached, no roof

overhead)

V□-BLDG-OTLN	Buildings and primary structures: outline

V□-BLDG-OVHD	Buildings and primary structures: overhead

V□-BLDG-PRCH	Buildings and primary structures: porch (attached, roof

overhead)

V□-BNDY	Political boundaries

V□-BNDY-BORO	Political boundaries: borough

V□-BNDY-CITY	Political boundaries: city

V□-BNDY-CNTY	Political boundaries: county

V□-BNDY-CORP	Political boundaries: corporation

V□-BNDY-NATL	Political boundaries: national

V□-BNDY-PROV	Political boundaries: province

V□-BNDY-STAT	Political boundaries: state

V□-BNDY-TSHP	Political boundaries: town or township

V□-BNDY-ZONE	Political boundaries: zoning

V□-BORE	Borings

V□-BRDG	Bridge

V□-BRDG-BENT	Bridge: top of bent

V□-BRDG-CNTR	Bridge: center

V□-BRDG-CTLJ	Bridge: control joint

V□-BRDG-DECK	Bridge: deck

V□-BRDG-GRAL	Bridge: guard rail

V□-BRKL	Break/fault lines

V□-BRKL-BOTB	Break/fault lines: bottom of bank

V□-BRKL-FLOW	Break/fault lines: flowline (lowest point of ditch) V□-BRKL-TOPB	 Break/fault lines: top of bank

V□-BRLN	Building restriction line

V□-BZNA	Buffer zone area

V□-CHAN	Navigable channels

V□-CHAN-BWTR	Navigable channels: breakwater

V□-CHAN-CNTR	Navigable channels: center

V□-CHAN-DACL	Navigable channels: de-authorized channel limits, anchorages,

etc.

V□-CHAN-DOCK	Navigable channels: decks, docks, floats, piers V□-CHAN-NAID	Navigable channels: navigation aids

V□-COMM	Communications

V□-COMM-MHOL	Communications: manhole

V□-COMM-OVHD	Communications: overhead

V□-COMM-POLE	Communications: pole

V□-COMM-UGND	Communications: underground

V□-CTRL	Control points

V□-CTRL-BMRK	Control points: benchmarks

V□-CTRL-FLYS	Control points: fly station

V□-CTRL-GRID	Control points: grid

V□-CTRL-HORZ	Control points: horizontal

V□-CTRL-HVPT	Control points: horizontal/vertical

V□-CTRL-PNPT	Control points: panel points

V□-CTRL-TRAV	Control points: transverse

V□-CTRL-VERT	Control points: vertical

V□-DRIV	Driveways

V□-DRIV-ASPH	Driveways: asphalt

V□-DRIV-CNTR	Driveways: center

V□-DRIV-CONC	Driveways: concrete

V□-DRIV-CURB	Driveways: curb

V□-DRIV-FLNE	Driveways: fire lane

V□-DRIV-GRVL	Driveways: gravel

V□-DRIV-MRKG	Driveways: pavement markings

V□-DRIV-UPVD	Driveways: unpaved surface

V□-DTCH	Ditches or washes

V□-DTCH-BOTM	Ditches or washes: bottom

V□-DTCH-CNTR	Ditches or washes: center

V□-DTCH-EWAT	Ditches or washes: edge of water

V□-DTCH-TOP~	Ditches or washes: top

V□-ESMT	Easements

V□-ESMT-ACCS	Easements: access (pedestrian only; private access)

V□-ESMT-CATV	Easements: cable television system

V□-ESMT-CONS	Easements: conservation

V□-ESMT-CSTG	Easements: construction/grading

V□-ESMT-ELEC	Easements: electrical

V□-ESMT-FDPL	Easements: flood plain

V□-ESMT-INEG	Easements: ingress/egress (vehicles; private access) V□-ESMT-LSCP	Easements: landscape

V□-ESMT-NGAS	Easements: natural gas line

V□-ESMT-PHON	Easements: telephone line

V□-ESMT-ROAD	Easements: roadway

V□-ESMT-ROAD-PERM	Easements: roadway: permanent V□-ESMT-ROAD-TEMP	Easements: roadway: temporary

V□-ESMT-RWAY	Easements: right-of-way (public access)

V□-ESMT-SGHT	Easements: sight distance

V□-ESMT-SSWR	Easements: sanitary sewer

V□-ESMT-STRM	Easements: storm sewer

V□-ESMT-SWMT	Easements: storm water management

V□-ESMT-TRAL	Easements: trail or path (public access)

V□-ESMT-UTIL	Easements: utility lines

V□-ESMT-WATR	Easements: water supply

V□-FLHA	Flood hazard area

V□-FUEL	Fuel systems

V□-FUEL-MHOL	Fuel systems: manhole

V□-FUEL-PIPE	Fuel systems: piping

V□-FUEL-TANK	Fuel systems: storage tanks

V□-FUEL-UGND	Fuel systems: underground

V□-NGAS	Natural gas systems

V□-NGAS-MHOL	Natural gas systems: manhole

V□-NGAS-PIPE	Natural gas systems: piping

V□-NGAS-TANK	Natural gas systems: storage tanks

V□-NGAS-UGND	Natural gas systems: underground

V□-NODE	Node

V□-NODE-ABUT	Node: abutment

V□-NODE-ACTL	Node: aerial horizontal and vertical control points V□-NODE-BLDG	Node: building points

V□-NODE-BLIN	Node: baseline

V□-NODE-BRDG	Node: bridge survey points

V□-NODE-BRKL	Node: break lines, spot elev. points and lines for creation of break lines as top of bank

V□-NODE-BROW	Node: brush row points

V□-NODE-BRSH	Node: brush points

V□-NODE-CABL	Node: underground cable systems

V□-NODE-CURB	Node: curb

V□-NODE-DASP	 Node: description attributes for survey points V□-NODE-DECK	Node: deck

V□-NODE-DRIV	Node: driveway

V□-NODE-EASP	Node: elevation attributes for survey points

V□-NODE-EXPJ	Node: expansion joint

V□-NODE-GRND	Node: ground

V□-NODE-MHOL	Node: manhole

V□-NODE-MRKG	Node: pavement markings (yellow/white stripes) V□-NODE-NGAS	Node: natural gas line

V□-NODE-PASP	 Node: point number attributes for survey points V□-NODE-PIPE	Node: piping (driveway/roadway culverts)

V□-NODE-POLE	Node: pole (power, telephone, etc.)

V□-NODE-PVMT	Node: pavement

V□-NODE-SIGN	Node: signage

V□-NODE-SSWR	Node: sanitary sewer

V□-NODE-STRM	Node: storm sewer

V□-NODE-SWLK	Node: sidewalks

V□-NODE-TREE	Node: tree

V□-NODE-TROW	Node: tree row

V□-NODE-WATR	Node: water supply

V□-POWR	Power

V□-POWR-FENC	Power: fences

V□-POWR-INST	Power: instrumentation (meters, transformers)

V□-POWR-MHOL	Power: manhole

V□-POWR-OVHD	Power: overhead

V□-POWR-POLE	Power: pole

V□-POWR-STRC	Power: structures

V□-POWR-UGND	Power: underground

V□-PRKG	Parking lots

V□-PRKG-ASPH	Parking lots: asphalt

V□-PRKG-CNTR	Parking lots: center

V□-PRKG-CONC	Parking lots: concrete

V□-PRKG-CURB	Parking lots: curb

V□-PRKG-DRAN	Parking lots: drainage slope indications

V□-PRKG-FLNE	Parking lots: fire lane

V□-PRKG-GRVL	Parking lots: gravel

V□-PRKG-MRKG	Parking lots: pavement markings

V□-PRKG-STRP	Parking lots: striping

V□-PRKG-UPVD	Parking lots: unpaved surface

V□-PROP	Property

V□-PROP-LINE	Property: lines

V□-PROP-QTRS	Property: quarter section

V□-PROP-RSRV	Property: reservation

V□-PROP-SBCK	Property: setback lines

V□-PROP-SECT	Property: section

V□-PROP-SUBD	Property: subdivision (interior) lines

V□-PROP-SXTS	Property: sixteenth section

V□-PVMT	Pavement

V□-PVMT-ASPH	Pavement: asphalt

V□-PVMT-CONC	Pavement: concrete

V□-PVMT-GRVL	Pavement: gravel

V□-RAIL	Railroad

V□-RAIL-CNTR	Railroad: center

V□-RAIL-EQPM	Railroad: equipment (gates, signals, etc.)

V□-RAIL-TRAK	Railroad: track

V□-RIVR	River

V□-RIVR-BOTM	River: bottom

V□-RIVR-CNTR	River: center

V□-RIVR-EDGE	River: edge

V□-RIVR-TOPB	River: top of bank

V□-ROAD	Roadways

V□-ROAD-ASPH	Roadways: asphalt

V□-ROAD-CNTR	Roadways: center

V□-ROAD-CONC	Roadways: concrete

V□-ROAD-CURB	Roadways: curb

V□-ROAD-FLNE	Roadways: fire lane

V□-ROAD-GRVL	Roadways: gravel

V□-ROAD-MRKG	Roadways: pavement markings

V□-ROAD-UPVD	Roadways: unpaved surface

V□-RRAP	Riprap

V□-RWAY	Right-of-way

V□-RWAY-CNTR	Right-of-way: center

V□-RWAY-CTLA	Right-of-way: controlled access

V□-RWAY-LINE	Right-of-way: lines

V□-RWAY-LMTA	Right-of-way: limited access

V□-RWAY-MRKR	Right-of-way: marker

V□-RWAY-STAN	Right-of-way: stationing

V□-SITE	Site features

V□-SITE-EWAT	Site features: edge of water

V□-SITE-FENC	Site features: fences

V□-SITE-ROCK	Site features: large rocks and rock outcroppings

V□-SITE-RTWL	Site features: retaining wall

V□-SITE-SIGN	Site features: signage

V□-SITE-VEGE	Site features: trees, shrubs, and other vegetation

V□-SSWR	Sanitary sewer

V□-SSWR-MHOL	Sanitary sewer: manhole

V□-SSWR-PIPE	Sanitary sewer: piping

V□-SSWR-STRC	Sanitary sewer: structures

V□-SSWR-UGND	Sanitary sewer: underground

V□-STEM	Steam system

V□-STEM-INST	Steam system: instrumentation (meters, valves, pumps) V□-STEM-MHOL	Steam system: manhole

V□-STEM-PIPE	Steam system: piping

V□-STEM-STRC	Steam system: structures

V□-STEM-UGND	Steam system: underground

V□-STRM	Storm sewer

V□-STRM-DTCH	Storm sewer: ditches or washes

V□-STRM-MHOL	Storm sewer: manhole

V□-STRM-PIPE	Storm sewer: piping

V□-STRM-POND	Storm sewer: retention pond

V□-STRM-STRC	Storm sewer: structures

V□-STRM-UGND	Storm sewer: underground

V□-SURV	Survey

V□-SURV-DATA	Survey: data

V□-SWLK	Sidewalks

V□-SWLK-ASPH	Sidewalks: asphalt

V□-SWLK-CONC	Sidewalks: concrete

V□-TOPO	Topographic feature

V□-TOPO-EWAT	Topographic feature: edge of water

V□-TOPO-GRID	Topographic feature: grid

V□-TOPO-MAJR	Topographic feature: major (contours)

V□-TOPO-MINR	Topographic feature: minor (contours)

V□-TOPO-SOUN	Topographic feature: soundings

V□-TOPO-SPOT	Topographic feature: spot elevations

V□-UNID	Unidentified site objects

V□-UNID-CABL	Unidentified site objects: cable systems

V□-UNID-PIPE	Unidentified site objects: piping

V□-UNID-TANK	Unidentified site objects: storage tanks

V□-UNID-UTIL	Unidentified site objects: utility lines

V□-UNID-UTIL-OVHD	 Unidentified site objects: utility lines: overhead V□-UNID-UTIL-UGND	 Unidentified site objects: utility lines: underground V□-WATR	Water supply

V□-WATR-INST	Water supply: instrumentation (meters, valves, pumps) V□-WATR-MHOL	Water supply: manhole

V□-WATR-PIPE	Water supply: piping

V□-WATR-STRC	Water supply: structures

V□-WATR-UGND	Water supply: underground

## TELECOMMUNICATIONS LAYER LIST

### Telecommunications Field Codes

The Layer Names shown below provide examples for the use of Major and Minor Group field codes for this discipline. See CLG Sections 1.5 and 1.6 for complete rules and options governing the use of Major and Minor Group field codes.

### Telecommunications Discipline Designators

#### Designator	Description

T	Telecommunications

TA	Audio Visual

TC	Clock and Program

TI	Intercom

TM	Monitoring

TN	Data Networks

TT	Telephone

TY	Security

TJ	User Defined

TK	User Defined

### Telecommunications Layer List

#### ayer Name	Description

T□-ALRM	Alarm system

T□-BCST	Broadcast-related system (radio or TV)

T□-BELL	Bell system

T□-CABL	Cable systems

T□-CABL-COAX	Cable systems: coax cable

T□-CABL-FIBR	Cable systems: fiber optics cable

T□-CABL-MULT	Cable systems: multi-conductor cable

T□-CABL-TRAY	Cable systems: cable tray and wireways

T□-CATV	Cable television system

T□-CCTV	Closed-circuit television system

T□-CLOK	Clock system

T□-CLOK-CIRC	Clock system: circuits

T□-CLOK-CLNG	Clock system: ceiling

T□-CLOK-CNMB	Clock system: circuit number

T□-CLOK-EQPM	Clock system: equipment

T□-CLOK-FLOR	Clock system: floor

T□-CLOK-WALL	Clock system: wall

T□-COMM	Communications

T□-COMM-CIRC	Communications: circuits

T□-COMM-CLNG	Communications: ceiling

T□-COMM-CNMB	Communications: circuit number

T□-COMM-EQPM	Communications: equipment

T□-COMM-FLOR	Communications: floor

T□-COMM-WALL	Communications: wall

T□-CONT	Controls and instrumentation

T□-CONT-DEVC	Controls and instrumentation: devices

T□-CONT-WIRE	Controls and instrumentation: wiring

T□-DATA	Data/LAN system

T□-DATA-CIRC	Data/LAN system: circuits

T□-DATA-CLNG	Data/LAN system: ceiling

T□-DATA-CNMB	Data/LAN system: circuit number

T□-DATA-EQPM	Data/LAN system: equipment

T□-DATA-FLOR	Data/LAN system: floor

T□-DATA-JACK	Data/LAN system: jacks

T□-DATA-WALL	Data/LAN system: wall

T□-DIAG	Diagrams

T□-DIAG-ENCL	Diagrams: equipment enclosures

T□-DIAG-EQPM	Diagrams: equipment

T□-DIAG-GRND	Diagrams: ground

T□-DICT	Dictation system

T□-DICT-CIRC	Dictation system: circuits

T□-DICT-CLNG	Dictation system: ceiling

T□-DICT-CNMB	Dictation system: circuit number

T□-DICT-EQPM	Dictation system: equipment

T□-DICT-FLOR	Dictation system: floor

T□-DICT-WALL	Dictation system: wall

T□-ELEC	Electrical system, telecom plan

T□-EMCS	Energy monitoring control system

T□-FIRE	Fire protection

T□-FIRE-CIRC	Fire protection: circuits

T□-FIRE-CLNG	Fire protection: ceiling

T□-FIRE-CNMB	Fire protection: circuit number

T□-FIRE-EQPM	Fire protection: equipment

T□-FIRE-FLOR	Fire protection: floor

T□-FIRE-WALL	Fire protection: wall

T□-INTC	Intercom/PA systems

T□-NURS	Nurse call system

T□-NURS-CIRC	Nurse call system: circuits

T□-NURS-CLNG	Nurse call system: ceiling

T□-NURS-CNMB	Nurse call system: circuit number

T□-NURS-EQPM	Nurse call system: equipment

T□-NURS-FLOR	Nurse call system: floor

T□-NURS-WALL	Nurse call system: wall

T□-PGNG	Paging system

T□-PHON	Telephone system

T□-PHON-JACK	Telephone system: jacks

T□-PROJ	Projector system

T□-SERT	Security system

T□-SERT-CIRC	Security system: circuits

T□-SERT-CLNG	Security system: ceiling

T□-SERT-CNMB	Security system: circuit number

T□-SERT-EQPM	Security system: equipment

T□-SERT-FLOR	Security system: floor

T□-SERT-WALL	Security system: wall

T□-SOUN	Sound system

T□-TRAN	Transmission system (RF and microwave)

T□-TVAN	Television antenna system

T□-TVAN-CIRC	Television antenna system: circuits

T□-TVAN-CLNG	Television antenna system: ceiling

T□-TVAN-CNMB	Television antenna system: circuit number

T□-TVAN-EQPM	Television antenna system: equipment

T□-TVAN-FLOR	Television antenna system: floor

T□-TVAN-WALL	Television antenna system: wall

T□-TVVS	Television and video systems

T□-TVVS-SAUD	Television and video systems: audio signal

T□-TVVS-SCOM	Television and video systems: communications sIgnal T□-TVVS-SCTL	Television and video systems: control signal

T□-TVVS-SDAT	Television and video systems: data signal

T□-TVVS-SDGA	Television and video systems: digital audio signal T□-TVVS-SDGV	Television and video systems: digital video signal T□-TVVS-SMIC	Television and video systems: microphone signal

T□-TVVS-SPWR	Television and video systems: power signal

T□-TVVS-SRFI	Television and video systems: RF signal

T□-TVVS-SRGB	Television and video systems: RGB and component video

signal

T□-TVVS-SSYN	Television and video systems: sync signal

T□-TVVS-SVID	Television and video systems: video signal

## OTHER DISCIPLINES LAYER LIST

### Other Disciplines Field Codes

The Layer Names shown below provide examples for the use of Major and Minor Group field codes for this discipline. See CLG Sections 1.5 and 1.6 for complete rules and options governing the use of Major and Minor Group field codes.

### Other Disciplines Discipline Designators

#### Designator	Description

X	Other Disciplines

XJ	User Defined

XK	User Defined

### Other Disciplines Layer List

#### Layer Name	Description

X□-RIGG	Other discipline: entertainment rigging/automation systems

X□-SPFX	Other discipline: entertainment special effects system

X□-VIDO	Other discipline: entertainment projection systems

# Appendix C - Complying with NCS and ISO 13567

## OVERVIEW

The International Standards Organization (ISO) is the only recognized international body promulgating standards in the area of electronic building design data. ISO Standard 13567, Organization and Naming of Layers for CAD, can be purchased at

http://www.ansi.org. The complete document is in three parts: 13567-1, 13567-2, and 13567-3.

While the United States National CAD Standard® (NCS) and ISO 13567 differ somewhat in their approach to standards for CAD layers, they are alike in several important respects. Both standards specify the names of the data fields that make up a typical layer name, define the field names, specify which fields are mandatory (required) and which fields are optional, specify the number of characters in each field, and specify the order in which the fields are to appear.

When one compares the NCS and ISO layer formats shown above, the question immediately arises whether it is possible to produce electronic building design documents that conform to both the NCS and ISO 13567. The answer is a qualified "yes."

Both standards provide several options for naming layers. The range of options allows either standard to meet the needs of diverse users and projects. By carefully choosing from among the available NCS options for naming CAD layers, and by establishing and adhering to the guidelines at the end of this Commentary, documents can be produced that are in full conformance with the

NCS and in conceptual conformance with ISO 13567 for the naming of CAD layers (an acceptable alternative to ISO default conformance). Adoption of the approach outlined herein could arguably reduce the effort required to produce documents in conformance with ISO 13567 by eliminating the ISO-mandated task of prescribing valid field codes for each project.

## FIELD CODES

| TWO STANDARDS OR ONE? |  |
| --- | --- |
| • | The NCS offers users an opportunity to comply with both U.S. and ISO CAD standards. By adhering to the guidelines in this commentary, summarized in ten (10) steps on the last page, the NCS becomes a "country-specific" implementation of the ISO CAD Standard. |
| • | For design firms doing international work, using the NCS can simplify the ISO-mandated task of preparing the layer naming system definition file that must accompany the project data files on every project. |

The NCS and ISO 13567 differ in one important respect. The NCS prescribes the valid alpha-numeric field codes that can appear in each data field, and the definitions of the field codes (e.g., EQPM = equipment). Users of ISO 13567 must determine, for each project, the valid field codes for that project and their definitions. ISO 13567 users are required to document this information in a metadata file known as a layer naming system definition file that must accompany the project data files. In its simplest form, this is nothing more than a tab-delimited text file.

There are valid reasons for both approaches. The prescriptive approach of the NCS relieves users of the task of developing and documenting field codes for every project. However, in order to accommodate all possible users, the list of prescribed NCS field codes must be comprehensive. By not prescribing field codes, ISO

13567 allows the ISO layer format to be applied uniformly without having to define all possible field codes in advance.

## FIELD CODES AND LANGUAGE

By not prescribing field codes, ISO 13567 also allows the ISO layer format to be applied uniformly without regard to language. Users may, if they wish, develop codes endowed with language-specific meaning. Citing our earlier example, English users might use the field code "EQPM" to represent the [major building] element "equipment," while users in another language group might use another field code that has similar mnemonic association to the word for "equipment" in that language.

While the field codes themselves might differ, the category of information contained in any given field is defined by the standard, facilitating translation of the actual content. In practice, ISO 13567 users tend to favor numeric codes to define the content of data fields. This eliminates any need to "translate" the field codes themselves. If, for example, the field code "720" is prescribed to mean "equipment," then only the definition, and not the code itself, would need to be translated. This eliminates the need for "translating" the actual file or layer name.

Default ISO Layer Format

(Mandatory Fields)

(Optional Fields)

ISO 13567 CONFORMANCE

| A | 1 | B | 2 | 1 | 0 | _ |  | D |  | N | B | 1 | 0 | 1 | 3 | 1 | F | R | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Agent Responsible |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

CAD data sets that adhere to ISO 13567 with respect to field names, field length, field definition and field order (as shown at right), and that are accompanied by the required layer naming system definition file, are defined by ISO to be in default conformance with the ISO standard.

| A | 1 | B | 2 | 1 | 0 | _ |  | D |  | N | B | 1 | 0 | 1 | 3 | 1 | F | R | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | [Building] Element |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| A | 1 | B | 2 | 1 | 0 | _ |  | D | _ | N | B | 1 | 0 | 1 | 3 | 1 | F | R | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Presentation |  |  |  |  |  |  |  |  |  |  |  |

ISO 13567 anticipates that groups of users or national standards bodies might not only wish to prescribe a list of valid field codes (as the NCS has done), but might also wish to vary from the specified ISO layer format. 13567-3 is explicitly designed "to allow national standards bodies (or projects where agreement is reached between the parties) to implement layer naming conventions which satisfy the requirements of the [ISO] standard while using alternative and more convenient layer naming structures and codes."

| A | 1 | B | 2 | 1 | 0 | _ |  | D | _ | N | B | 1 | 0 | 1 | 3 | 1 | F | R | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  | Status |  |  |  |  |  |  |  |  |  |

| A | 1 | B | 2 | 1 | 0 | _ |  | D | _ | N | B | 1 | 0 | 1 | 3 | 1 | F | R | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  | Sector |  |  |  |  |  |  |  |  |

To permit this, ISO 13567-3 establishes rules for modifying the layer format itself. As with the field codes, users are required to fully document layer format modifications in the layer naming system definition file. CAD data sets that adhere to these rules are defined by ISO as being in conceptual conformance with the ISO standard, an approved alternative to default conformance.

| A | 1 | B | 2 | 1 | 0 | _ |  | D | _ | N | B | 1 | 0 | 1 | 3 | 1 | F | R | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Phase |  |  |  |  |

| A | 1 | B | 2 | 1 | 0 | _ |  | D | _ | N | B | 1 | 0 | 1 | 3 | 1 | F | R | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Projection |  |  |  |

| A | 1 | B | 2 | 1 | 0 | _ |  | D | _ | N | B | 1 | 0 | 1 | 3 | 1 | F | R | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Scale |  |  |

The rules for conceptual conformance specify that the mandatory data fields must always be used, but the order of all fields in the layer name (both mandatory and optional), the number of optional fields used, and the number of characters in each field can vary from the default ISO layer format. Additionally, the names of the fields can differ from the names specified, as long as the conceptual definition of each field conforms to the ISO standard. All modifications to the default layer format must be applied uniformly throughout the project. Layer names must all be of the same length, use the same set of mandatory and optional fields in the same order, and have the same number of characters per field.

| A | 1 | B | 2 | 1 | 0 | _ |  | D | _ | N | B | 1 | 0 | 1 | 3 | 1 | F | R | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | Work Package |  |  |  |  |  |

These rules allow data sets created in conceptual conformance with ISO 13567 to be mapped to the ISO 13567 default layer format.

However, ISO does not require users to actually "map" or otherwise convert the data into the default layer format.

## FIELD NAMES AND DEFINITIONS

Though the specified field names in the NCS layer format differ from the specified field names in the ISO layer format, the definitions of the field names are conceptually the same (with one important exception, discussed in the next paragraph below). This allows NCS-compliant data to meet the principal ISO 13567 criterion for conceptual conformance. The Field Name Comparison Table at right highlights additional rules that must be followed to create data that is in conformance with both the NCS and ISO 13567.

## DISCIPLINE DESIGNATOR" VS. "AGENT RESPONSIBLE"

| Field Name Comparison Table |  |
| --- | --- |
| NCS Field Name | ISO Field Name |
| Discipline Designator | Agent Responsible |
| Major & Minor Groups | Element |
| Annotation Minor Group* | Presentation |
| Status** | Status |
| (none) | Sector |
| Status (Phase)** | Phase |
| Dwg. View Minor Group*** | Projection |
| (none) | Scale |
| (none) | Work Package |

The conceptual definitions of these corresponding field names in the NCS and ISO 13567 differ sufficiently to merit detailed discussion. The definition for Discipline Designator is defined in NCS as "the category of subject matter contained in the file or layer designated." In other words, if the information contained is "structural," the file or layer name will begin with the Discipline Designator "S," regardless of who created the data.

ISO 13567 defines Agent Responsible as "the construction specialist responsible for the data." Regrettably, ISO 13567 does not further define the terms "construction specialist" and "responsible for."

"Construction specialist" could be interpreted to mean "design professional," "design drafter," or even "skilled tradesperson or contractor." Though the text of ISO 13567 does not define which of these individuals is the "agent responsible," one can reasonably infer from the sample layer naming system definition file shown in Annex A of ISO 13567-3 that "construction specialist" is defined as the design professional.

## "AGENT RESPONSIBLE" AND PROFESSIONAL LIABILITY

Identifying the design professional as the "construction specialist" still allows considerable room for interpretation of the definition for

* ISO compliance requires that the last NCS Minor Group field be reserved for annotation.

** ISO compliance requires that this field be reserved for status OR project phase, but not both; duplicate use of the field is not permitted.

*** ISO compliance requires that Drawing View field names not appear in the same fields as Major or Minor Group fields that define major building elements. If both annotation and drawing view are to be included in any layer names, one Minor Group Field must be reserved for Annotation and the other for Drawing View.

"agent responsible." It could be interpreted to mean either "design professional who is professionally liable for the information by virtue of professional licensure and role on the project," or, alternatively, "design professional who is professionally liable for the information by virtue of having signed and sealed the document in question." An example is a lighting plan prepared under the supervision of, and signed and sealed by, the architect. Should the field code for this drawing file or layer name be "E" or "A?" If the field code is "E," is the Electrical Engineer still the designated "Agent Responsible," and therefore professionally liable for data created by others not under his/her supervision?

The burden of professional liability borne by design professionals is generally less in other countries than it is in the

U.S. Perhaps for this reason, the issue of defining agent responsible more precisely with respect to professional liability did not arise when this field name was defined by ISO 13567.

In the U.S., however, use of the imprecise ISO definition for agent responsible might possibly expose design professionals to professional liability for data over which they had no oversight.

## "DISCIPLINE DESIGNATOR" AND THE BUILDING LIFE CYCLE

The NCS definition for the field Discipline Designator was agreed-upon following considerable debate by the NCS Project Committee, and with the full understanding that it differed from the conceptual definition of the corresponding ISO 13567 field Agent Responsible. In addition to the liability issues cited above, it was the consensus of the Project Committee that the ability to identify the data by subject matter throughout the life-cycle of a building facility was ultimately more important than the identity of the person or persons who originally created the data.

## "DISCIPLINE DESIGNATOR" AND ISO 13567 CONFORMANCE

The difference in the conceptual definitions of Discipline Designator and Agent Responsible would seem to be an insurmountable obstacle to creating data in conformance with both the NCS and ISO 13567. This is not necessarily true. In most cases, the content of the fields Discipline Designator and Agent Responsible are one and the same, regardless of the definition. For example, if the subject matter contained in the drawing file or layer is "mechanical systems," the mechanical engineer is likely to be the design professional under whose supervision the data was created.

Users who wish to produce data that is in conformance with the NCS and in conceptual conformance with ISO 13567 can do so by establishing a rule for their projects that data will be created only under the supervision of the design professional typically responsible for the subject matter. In this way, the conceptual definition for the data field can be BOTH "category of subject matter contained in the file or layer designated" AND "construction specialist responsible for the data." Implementation of this rule can help reduce the risk of professional liability by minimizing the likelihood of conflicts that might arise when different elements of the same building system are designed by more than one design professional.

## FIELD CODE RESTRICTIONS

A key principal of the ISO 13567 layer format is that each data field can be used to define only one category of data. Duplicate use of a field is prohibited. This ensures that data sets in conceptual conformance can be readily mapped to the ISO default layer format. Adherence to this provision requires NCS users to restrict their use of certain NCS field codes.

The NCS allows "ANNO" to be used as a Major Group, which allows all annotation to be placed in a defined group of layers. This results in a duplicate use of the Major Group field. The corresponding field in ISO, "Element," is reserved for major building elements. Therefore, the field code "ANNO" cannot be used at all CLG Figure 6.10-1. However, the prescribed annotation Minor Group field codes (TEXT, DIMS, etc.) can be used to modify any preceding Major/Minor Group, provided that the field in which they appear is reserved for annotation field codes.

If Drawing View field codes are used CLG Figure 6.10-2, the Minor Group field in which they appear must likewise exclude any other field codes.

If the Status field is used CLG Figure 6.10-3, the allowable field codes must be restricted to the specified letters (to correspond to the ISO field "Status") or to the specified numbers (to correspond to the ISO field "Phase") but not both.

U.S. NCS Field Code Restrictions

(for conceptual conformance to ISO 13567)

The field code "ANNO" may NOT be used, because "annotation" is not a major building "element:"

| A | I | - | A | N | N | O | - | T | E | X | T | - | N |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Major Group (Element) |  |  |  |  |  |  |  |  |  |  |

The Annotation Minor Group field codes MAY be used, provided the field is reserved for these codes. Two allowable formats are shown:

| A | I | - | W | A | L | L | - | T | E | X | T | - | N |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | (Presentation) Annotation |  |  |  |  |  |  |  |  |  |  |

| A | I | - | W | A | L | L | - | F | U | L | L | - | T | E | X | T | - | N |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | (Presentation) Annotation |  |  |  |  |  |  |  |  |  |  |

CLG Figure 6.10-1 - Annotation Field Codes

Two allowable formats for Drawing View field codes:

| A | I | - | W | A | L | L | - | E | L | E | V | - | N |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | (Projection) Drawing View |  |  |  |  |  |  |  |  |  |  |

| A | I | - | W | A | L | L | - | F | U | L | L | - | E | L | E | V | - | N |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | (Projection) Drawing View |  |  |  |  |  |  |  |  |  |  |

CLG Figure 6.10-2 - Drawing View Field Codes

Two allowable formats for Status field codes.

| A | - | W | A | L | L | - | E | L | E | V | - | T | E | X | T | - | N |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  | (Status) Status |  |  |  |  |  |

| A | - | W | A | L | L | - | E | L | E | V | - | T | E | X | T | - | 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  | (Phase) Status |  |  |  |  |  |

CLG Figure 6.10-3 - "Status" Field Codes

Example NCS Layer Formats

(in conceptual confomance to ISO 13567)

CLG Figure 6.11-1

CLG Figure 6.11-2

NCS AND ISO 13567 IMPLEMENTATION OPTIONS

| A | I | - | W | A | L | L | - | F | U | L | L | - | T | E | X | T | - | N |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Level 2 Discipline Designator (Agent Responsible) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

The examples shown here illustrate two possible NCS layer formats that are in conceptual conformance with ISO 13567. Note that ISO 13567 does not use dashes as field delimiters. For purposes of ISO conformance, the dashes in the NCS layer format are defined as an additional character of the field preceding it.

| A | I | - | W | A | L | L | - | F | U | L | L | - | T | E | X | T | - | N |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Major, Minor Group (Element) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| A | I | - | W | A | L | L | - | F | U | L | L | - | T | E | X | T | - | N |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | (Presentation) Annotation |  |  |  |  |  |  |  |  |  |  |

| A | I | - | W | A | L | L | - | F | U | L | L | - | T | E | X | T | - | N |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  | (Status) Status |  |  |  |  |  |

CLG Figure 6.11-1 shows the optional two-character NCS Level 2 Discipline Designator; together with the dash that follows it, this field is defined as three (3) characters in length. A Major and one Minor Group are defined as corresponding to the ISO field [Building] "Element." The field is ten (10) characters in length. The second Minor Group is reserved for Annotation field codes, corresponds to the ISO field "Presentation," and is five (5) characters in length. The final field is Status, which corresponds to the ISO field of the same name, and is one (1) character in length.

| A | - | W | A | L | L | - | E | L | E | V | - | T | E | X | T | - | 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Level 1 Discipline Designator (Agent Responsible) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| A | - | W | A | L | L | - | E | L | E | V | - | T | E | X | T | - | 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Major Group (Element) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| A | - | W | A | L | L | - | E | L | E | V | - | T | E | X | T | - | 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Drawing View (Projection) |  |  |  |  |  |  |  |  |  |  |

CLG Figure 6.11-2 shows the NCS required Level 1 Discipline Designator only, and is defined as two (2) characters in length. The Major Group is defined as corresponding to the ISO field [Building] "Element," and is five (5) characters in length. The first Minor Group is reserved for Drawing View field codes, corresponds to the ISO field "Projection," and is five (5) characters in length. The second Minor Group is reserved for Annotation field codes, corresponds to the ISO field "Presentation," and is five (5) characters in length. The final field is reserved for Phase field codes, corresponds to the ISO field "Phase," and is one (1) character in length.

| A | - | W | A | L | L | - | E | L | E | V | - | T | E | X | T | - | 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | (Presentation) Annotation |  |  |  |  |  |  |  |  |  |  |

Note that for ISO conformance, the total length of the layer name must be the same for all layers on a given project. Layer names that do not require a certain field, such as "Annotation," must use placeholders (usually dashes or underscores) to maintain the length of the layer name and the relative position of the fields.

| A | - | W | A | L | L | - | E | L | E | V | - | T | E | X | T | - | 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  | (Phase) Status |  |  |  |  |  |

While the ISO 13567 rules for conceptual conformance allow the fields to appear in any order, this is not permitted by the NCS. The fields must be in the order of Discipline Designator, Major Group, Minor Group 1, Minor Group 2, Status. If a Minor Group field is used to modify the "building element" shown in the Major Group, that Minor Group must appear immediately following the Major Group.

## NCS AND ISO 13567 IMPLEMENTATION GUIDELINES

The information in this Commentary is summarized in the following steps for preparing documents with layer names in conformance with the NCS and in conceptual conformance with ISO 13567. While these guidelines are intended to aid NCS users, adherence to these rules in some form would be required by ISO 13567 whether or not the NCS layer format were used.

    - Require that all documents be prepared only under the supervision of the design professional typically responsible for the subject matter contained in the documents.

    - Do not use the field code "ANNO" in any layer name.

    - Determine whether the Discipline Designator will be one character (Level 1) or two characters (Level 2) in length.

    - Determine whether the "building element" will consist of a Major Group only, or of a Major Group and one Minor Group.

    - Determine whether a Minor Group is to be reserved for Drawing View field codes, and fix its position in the sequence of fields.

Required Use of Placeholders

(for conceptual conformance to ISO 13567)

Layers in which reserved field codes are not used must have placeholders in the reserved fields.

CLG Figure 6.12-1

Determine whether a Minor Group is to be reserved for Annotation field codes, and fix its position in the sequence of fields.

| A | - | W | A | L | L | - | E | L | E | V | - | _ | _ | _ | _ | - | N |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Note that only two Minor Groups are available. Of the three options described in 4, 5, and 6 above, only two can be exercised on a given project.

| A | - | W | A | L | L | - | E | L | E | V | - | T | E | X | T | - | N |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Determine whether to include the Status field in the layer name and whether to use the specified letters to denote "Status," or the specified numbers to denote "Phase."

    - For layer names in which one or more fields are not required,

use placeholders (dashes or underscores) to maintain consistent layer name length and the relative positions of fields. Refer to CLG Figure 6.12-1.

    - Prepare a layer naming system definition file in accordance with ISO 13567-3 that defines the selected layer format for the project.

National Institute of Building Sciences | An Authoritative Source of Innovative Solutions for the Built Environment 1090 Vermont Avenue, NW, Suite 700 | Washington, DC 20005-4950 | (202) 289-7800 | Fax (202) 289-1092

© 2011 National Institute of Building Sciences. All rights reserved.